edge to the cervical point is 8.7 mm and thus the root length 11.8 mm.

Table 2 Crown designs for the lateral incisor. NoAt: Control design without any attachment; recV: Ideal vertical rectangular attachment. recVf: Vertical rectangular attachment with excessive material at the base. recVfs: Vertical rectangular attachment with excessive material at the base and shifted; bevC: Beveled attachment in the center of the facial crown surface. bevO: Beveled attachment occlusally shifted. bevG: Beveled attachment shifted towards the gingival margin

| NoAt | No attachment |  | Facial-lingual view | Distal-mesial view |
| --- | --- | --- | --- | --- |
| recV | Position | Center | 1.5 mm 3 mm | 1 mm |
|  | Height | 3 mm |  |  |
|  | Depth | 1 mm |  |  |
|  | Width | 1.5 mm |  |  |
|  | Beveling angle | 45° |  |  |
|  | Edge blend radius | 0.2 mm |  |  |
|  | Disc radius | None |  |  |
|  | Disc depth | None |  |  |
| recVf | Position | Center | +1 mm | 1.2…1.5 mm |
|  | Height | 3 mm |  |  |
|  | Depth | 1 mm |  |  |
|  | Width | 1.5 mm |  |  |
|  | Beveling angle | 45° |  |  |
|  | Edge blend radius | 0.2 mm |  |  |
|  | Disc radius | 2 mm |  |  |
|  | Disc depth | 0.3 mm |  |  |
| recVfs | Position | 1 mm shift to incisal edge |  |  |
|  | Height | 3 mm |  |  |
|  | Depth | 1 mm |  |  |
|  | Width | 1.5 mm |  |  |
|  | Beveling angle | 45° |  |  |
|  | Edge blend radius | 0.2 mm |  |  |
|  | Disc radius | 2 mm |  |  |
|  | Disc depth | 0.3 mm |  |  |
| bevC | Position | Center |  |  |
|  | Height | 2.5 mm |  |  |
|  | Depth | 1.5 mm |  |  |
|  | Width | 3.0 mm |  |  |
|  | Beveling angle | 33.69° |  |  |
|  | Edge blend radius | 0.2 mm |  |  |
| bevO | Position | 2 mm occlusal shift | +2 mm |  |
|  | Height | 2.5 mm |  |  |
|  | Depth | 1.5 mm |  |  |
|  | Width | 3.0 mm |  |  |
|  | Beveling angle | 33.69° |  |  |
|  | Edge blend radius | 0.2 mm |  |  |
| bevG | Position | 2 mm apical shift | -2 mm |  |
|  | Height | 2.5 mm |  |  |
|  | Depth | 1.5 mm |  |  |
|  | Width | 3.0 mm |  |  |
|  | Beveling angle | 33.69° |  |  |
|  | Edge blend radius | 0.2 mm |  |  |