Supporting Information

ASSESSMENT OF SNOWMELT QUALITY DISCHARGING FROM A COLD-CLIMATE URBAN LANDSCAPE DURING SPRING MELT

Hayley Popick1, Markus Brinkmann2,3,4,5, and Kerry McPhedran 1,2*

1 Department of Civil, Geological, and Environmental Engineering, College of Engineering, University of Saskatchewan, Saskatoon, SK, Canada

2 Global Institute for Water Security, University of Saskatchewan, Saskatoon, SK, Canada

3 Toxicology Centre, University of Saskatchewan, Saskatoon, SK, Canada

4 School of Environment and Sustainability, University of Saskatchewan, Saskatoon, SK, Canada

5 Centre for Hydrology, University of Saskatchewan, Saskatoon, SK, Canada

Pages: 14

Tables: 11

Figures: 1

#### Table S1. Snow season leading up to 2019 and 2020 snow sampling seasons. Monthly snowfall values were recorded at the CRS (Wittrock 2020). Cumulative normals begin in January and do not reflect seasonal accumulation. It is assumed that the entire depth of snow falling on roadways was transported to a snow storage facility. Snowfall is expressed as snow-water equivalent (SWE).

|  | Snowfall Months | Precipitation (SWE, mm) | 1981-2010 climate normals | Cumulative 2019 | % of cumulative normal |
| --- | --- | --- | --- | --- | --- |
| 2018 | November | 12.9 | 13.4 | 213.1 | 62.2 |
|  | December | 3.2 | 12.7 | 216.3 | 60.9 |
| 2019 | January | 8.4 | 15.5 | 8.4 | 54.2 |
|  | February | 14.2 | 9.3 | 22.6 | 91.1 |
|  | March | 3.9 | 13.8 | 26.5 | 68.7 |
|  | April | 2.9 | 22.9 | 29.4 | 47.8 |
| Seasonal Total |  | 45.5 |  |  |  |
|  | Snowfall Months | Precipitation (SWE, mm) | 1981-2010 climate normals | Cumulative 2020 | % of cumulative normal |
| 2019 | October | 7 | 19.2* | 275.5 | 83.7 |
|  | November | 14.3 | 13.4 | 289.8 | 84.6 |
|  | December | 6.1 | 12.7 | 295.9 | 83.3 |
| 2020 | January | 9.7 | 15.5 | 9.7 | 62.6 |
|  | February | 3.5 | 9.3 | 13.2 | 53.2 |
|  | March | 12.5 | 13.8 | 25.7 | 66.6 |
|  | April | 10.6 | 22.9 | 40.5 | 65.9 |
| Seasonal Total |  | 63.7 |  |  |  |

#### Table S2. Snowmelt sampling events from snow storage facilities within the City of Saskatoon. Facilities were selectively sampled; sampling events are noted with an asterisk. Both snow from the pile and snowmelt were collected.

|  |  | Snow Storage Facility |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Date | Valley Rd. (52.101, -106.736) | USask (52.137, -106.636) | Wanuskewin Rd. (52.219, -106.626) | Central Ave. (52.187, -106.592) |
| 2019 | April 2nd |  | *** |  |  |
|  | April 13th | * | * | * |  |
|  | April 18th | * |  | * | * |
|  | April 24th | * | * | * | * |
| 2020 | March 7th | *** | *** |  |  |
|  | March 26th | ** |  |  |  |
|  | March 29th | ** |  |  |  |
|  | April 10th | ** |  |  |  |
|  | April 17th | ** | ** | ** | ** |
|  | April 20th | ** |  |  |  |
|  | April 23rd | ** | ** | ** | ** |
|  | April 28th | ** | ** |  | ** |
|  | May 1st | ** | ** | ** |  |
|  | May 5th | ** | ** |  | ** |
|  | May 12th | ** | ** | ** |  |

*SP samples collected only.
**Snowmelt samples collected only.
***Both SP and snowmelt samples collected.

Table S3. Limits of detection (LOD) and quantification (LOQ) for polyaromatic hydrocarbons.

| Sample | Acenaphthylene | Fluorene | BbF | BkF | BaP | Benzo[a] anthracene | Chrysene | Dibenzo[a,h] anthracene | Pyrene | Phenanthrene | Antrhacene | Indeno[123]cd-pyrene | Benzo[ghi] perylene |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LOD (ppb) | 0.004 | 0.002 | 0.004 | 0.006 | 0.006 | 0.002 | 0.004 | 0.012 | 0.004 | 0.004 | 0.004 | 0.004 | 0.006 |
| LOQ (ppb) | 0.02 | 0.02 | 0.026 | 0.034 | 0.032 | 0.014 | 0.022 | 0.074 | 0.022 | 0.02 | 0.022 | 0.03 | 0.042 |

Table S4: Land use classification breakdowns of City of Saskatoon (CoS) catchments. Land use delineations are adapted from Al Masum et al. (2021). Study catchments drain to the sampled SW outfalls. Represented as percentage of land use over total area of the CoS, as shown by labelled catchments in Figure S1.

| Catchment Name | Area (km2) | CoS (%) | Single Residential (%) | Multi Residential (%) | Roads (%) | Highways (%) | Commercial (%) | Industrial (%) | Green/ Parks (%) | Agriculture (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Circle Dr S Bridge | 24.6 | 10.8 | 35 | 10 | 9 | 7 | 7 | 15 | 17 | 0 |
| Light & Power | 9.58 | 4.20 | 25 | 5 | 8 | 8 | 9 | 25 | 20 | 0 |
| 17th St W | 9.27 | 4.06 | 39 | 16 | 8 | 5 | 14 | 5 | 5 | 0 |
| Taylor St | 8.47 | 3.71 | 71.26 | 2.44 | 10 | 5 | 1.24 | 2.22 | 5.78 | 0.92 |
| 14th St E | 4.27 | 1.87 | 54 | 12 | 9 | 4 | 10 | 0 | 8 | 3 |
| Avenue B S | 1.20 | 0.53 | 18 | 2 | 8 | 7 | 62 | 0 | 3 | 0 |
| 23rd St W | 2.73 | 1.20 | 20 | 6 | 10 | 6 | 29 | 25 | 4 | 0 |
| Weir/33rd St | 10.05 | 4.40 | 12 | 3 | 4 | 2 | 26.79 | 45.21 | 6 | 0 |
| Preston Crossing | 33.23 | 14.57 | 47.26 | 7.76 | 4.4 | 5.21 | 10.14 | 0.75 | 3.86 | 15 |
| Spadina/ Sturgeon | 7.03 | 3.08 | 35 | 3 | 3 | 2 | 10 | 41 | 6 | 0 |
| Preston Dog Park | 26.81 | 11.75 | 17 | 3 | 2 | 0.5 | 2 | 0 | 65.56 | 3 |
| Whiteswan/WWTP | 3.92 | 1.72 | 64 | 5 | 3 | 0.5 | 0 | 0 | 26.5 | 0 |
| Wanuskewin Rd. | 25.5 | 11.2 | 0 | 0 | 4 | 4 | 0 | 37 | 55 | 0 |
| TOTAL | 76.3 | 33.5 | 29.9 | 5.42 | 5.38 | 4.17 | 7.62 | 14.1 | 26.9 | 3.74 |

#### Table S5. Seasonal snow results by site. The high standard deviation (SD) in USask and Valley Rd. samples is due to averaging SP and snowmelt data from the same sampling event (April 2/19 and March 7/20, respectively).

| Sampling Year and Site | Events | pH | TDS (mg/L) | EC (μS/cm) | DOC (ppm) | COD (mg/L) | TSS (mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2019 |  |  |  |  |  |  |  |
| Valley Rd. | 3 | 9.34 (0.06) | 26.1 (2.8) | 55.7 (5.7) | 1.17 (0.19) | 497 (193) | 1,602 (833) |
| USask Campus | 3 | 8.63 (0.67) | 94.4 (129) | 196 (263) | 4.45 (1.75) | 407 (248) | 2,107 (1,815) |
| Wanuskewin Rd. | 3 | 9.28 (0.19) | 26.7 (2.1) | 56.2 (3.5) | 1.71 (1.30) | 389 (249) | 1,634 (720) |
| Central Ave. | 2 | 9.34 (0.09) | 23.4 (1.06) | 50.0 (2.2) | 1.24 (0.43) | 447 (6.7) | 1,060 (140) |
| Average (SD) |  | 9.15 (0.25) | 42.6 (33.6) | 89.4 (68.6) | 2.14 (0.92) | 435 (174) | 1,601 (877) |
| 2020 |  |  |  |  |  |  |  |
| Valley Rd. | 11 | 8.06 (0.31) | 2,533 (2538) | 4,723 (4,498) | 9.45 (7.98) | 260 (152) | 351 (669) |
| USask Campus | 7 | 7.99 (0.50) | 608 (342) | 1,220 (668) | 4.43 (4.39) | 190 (127) | 770 (642) |
| Wanuskewin Rd. | 4 | 8.13 (0.07) | 251 (47.0) | 518 (95.0) | 1.38 (1.75) | 154 (89.1) | 257 (58) |
| Central Ave. | 4 | 7.73 (0.37) | 2,020 (948) | 3,881 (1,764) | 8.03 (1.95) | 308 (150) | 1,078 (1,278) |
| Average (SD) |  | 7.83 (2.72) | 1,133 (877) | 2,189 (1,636) | 6.91 (5.71) | 212 (146) | 462 (493) |

#### Table S6. Full 2019 sampling results. The sampling window occurred over April 2019; melt events were observed for two weeks prior to sample. Snow pile samples were collected as aggregate samples from 8-12 points on on-site snow piles and melted in the Usask Environmental Engineering labs prior to analysis.

| Sampling Site Name and Date | pH | TDS (mg/L) | EC (μS/cm) | DOC (mg/L) | COD (mg/L) | TSS (mg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| April 2 2019 |  |  |  |  |  |  |
| USask Campus (SM)* | 7.66 | 590 | 287 | 5.42 | 246 | 573 |
| USask Campus (SP)* | 8.76 | 51.3 | 24 | 1.95 | 158 | 506 |
| AVERAGE (SD) | 8.21 (0.78) | 156 (186) | 321 (381) | 3.68 (2.45) | 202 (62.6) | 539.5 |
| April 13 2019 |  |  |  |  |  |  |
| USask Campus | 8.94 | 72 | 34 | 4.55 | 690 | 3,512 |
| Valley Rd | 9.29 | 60.2 | 28.3 | 1.07 | 685 | 2,561 |
| Wanuskewin Rd | 9.07 | 60.2 | 29.1 | 3.21 | 676 | 2134 |
| AVERAGE (SD) | 9.10 (0.18) | 30.5 (3.1) | 64.1 (6.8) | 2.94 (1.75) | 684 (7.4) | 2,736(706) |
| April 18 2019 |  |  |  |  |  |  |
| Valley Rd | 9.4 | 57.6 | 27 | 1.39 | 300 | 1180 |
| Wanuskewin Rd | 9.43 | 54.9 | 25.8 | 1.01 | 231 | 1,958 |
| Central Ave | 9.4 | 51.5 | 24.1 | 0.94 | 443 | 1,158 |
| AVERAGE (SD) | 9.41 (0.02) | 25.6 (1.5) | 54.7 (3.1) | 1.11 (0.25) | 324 (108) | 1,432 |
| April 24 2019 |  |  |  |  |  |  |
| USask Campus | 9.16 | 68.9 | 32.5 | 5.87 | 533 | 3,838 |
| Valley Rd | 9.33 | 49.2 | 23 | 1.05 | 505 | 1,065 |
| Wanuskewin Rd | 9.33 | 53.6 | 25.1 | 0.91 | 262 | 809 |
| Central Ave | 9.27 | 48.4 | 22.6 | 1.55 | 452 | 961 |
| AVERAGE (SD) | 9.27 (0.08) | 25.8 (4.6) | 55.0 (9.5) | 2.35 (2.36) | 438 (122) | 1,668(1,450) |

*Sampled from meltwater pools. All other 2019 samples were collected from various locations and depths on the SP.

#### Table S7. Full 2020 SM quality results. Snow was sampled during March-May 2020. No significant melting was observed prior to the sampling season. Snowmelt was collected from pools of runoff located onsite. Note the samples from Valley Rd. were collected from an effluent stream draining to a designated SM outlet surface; outlets are not present at the other sites.

| Sampling Site Name and Date | pH | TDS (mg/L) | EC (μS/cm) | DOC (mg/L) | COD (mg/L) | TSS (mg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| March 7 2020 |  |  |  |  |  |  |
| Valley Rd (SP)** | 8.12 | 1,656 | 827 | 8.58 | 259.5 | 2,457 |
| Valley Rd (SM) | 7.72 | 5,850 | 3,080 | 20.39 | 238.5 | 162 |
| USask Campus (SP)** | 7.56 | 2,180 | 1,100 | 8.42 | 285 | 1,139 |
| USask Campus (SM) | 7.67 | 1,560 | 778 | 10.76 | 182 | 1,257 |
| AVERAGE (SD) | 7.77 (0.24) | 1,446 (1,098) | 2,812 (2,044) | 12.04 (5.67) | 241 (43.8) | 1,253 (940) |
| March 26 2020 |  |  |  |  |  |  |
| Valley Rd. | 7.75 | 15,370 | 8,650 | 27.35 | 645.5 | 40.5 |
| March 29 2020 |  |  |  |  |  |  |
| Valley Rd. | 8.41 | 2,620 | 1,331 | 6.12 | 231 | 312 |
| April 10 2020 |  |  |  |  |  |  |
| Valley Rd. | 8.63 | 2,370 | 1,200 | 3.82 | 138 | 225 |
| April 17 2020 |  |  |  |  |  |  |
| USask | 7.2 | 837 | 409 | 5.83 | 448 | 1,800 |
| Valley Rd. | 7.56 | 9,570 | 5,200 | 17.14 | 410 | 31 |
| Wanuskewin Rd. | 8.06 | 506 | 245 | 3.40 | 116.5 | 219 |
| Central Ave. | 7.26 | 5,360 | 2,820 | 9.43 | 436 | 656 |
| Light & Power SW** | 7.84 | 1,637 | 816 | 9.97 | 116 | 1 |
| 23rd St E SW** | 7.24 | 1,157 | 570 | 8.23 | 242 | 312 |
| AVERAGE (SD) | 7.53 (0.36) | 1,677 (1,967) | 3,178 (3,597) | 9.00 (4.68) | 295 (157) | 503 (678) |
| April 20 2020 |  |  |  |  |  |  |
| Valley Rd. | 8 | 1,949 | 981 | 6.04 | 193 | 131 |
| April 23 2020 |  |  |  |  |  |  |
| USask | 8.18 | 993 | 486 | 0.72 | 147 | 364 |
| Valley Rd. | 8.25 | 9,380 | 5,110 | 8.36 | 311.5 | 75 |
| Wanuskewin Rd. | 8.08 | 533 | 258 | 0.34 | 85.5 | 229 |
| Central Ave. | 7.65 | 5,050 | 2,650 | 7.29 | 422 | 302 |
| AVERAGE (SD) | 8.04 (0.27) | 2,126 (2,263) | 3,989 (4,127) | 4.18 (4.24) | 242 (154) | 242 (124) |
| April 28 2020 |  |  |  |  |  |  |
| USask Campus | 8.6 | 2,050 | 1,040 | 0.68 | 87 | 416 |
| Valley Rd | 8 | 2,740 | 1,395 | 1.95 | 287 | 234 |
| Central Ave | 7.88 | 3,620 | 1,868 | 5.63 | 255 | 2982 |
| AVERAGE (SD) | 8.16 (0.39) | 1,434 (415) | 2,803 (787) | 2.30 (3.17) | 210 (107) | 1,210 (1,537) |
| May 1st 2020 |  |  |  |  |  |  |
| USask Campus | 8.36 | 325 | 157 |  | 89.5 |  |
| Valley Rd | 7.94 | 3,350 | 1721 | 7.62 | 199 | 244 |
| Wanuskewin Rd | 8.18 | 401 | 194 |  | 285 |  |
| AVERAGE (SD) | 8.15 (0.21) | 1,104 (893) | 2,159 (1,725) |  | 97.7 (166) |  |
| May 5th 2020 |  |  |  |  |  |  |
| USask Campus | 8.14 | 1,452 | 721 | 4.86 | 193 | 153 |
| Valley Rd | 8.29 | 276 | 133 | 4.01 | 60 | 216 |
| Central Ave | 8.14 | 1,492 | 741 | 9.78 | 120 | 373 |
| AVERAGE (SD) | 8.19 (0.09) | 532 (346) | 1,073 (282) | 6.22 (3.12) | 124 (66.4) | 247 (114) |
| May 12th 2020 |  |  |  |  |  |  |
| USask Campus | 8.23 | 363 | 175 | 1.07 | 92 | 263 |
| Valley Rd | 8.08 | 1,541 | 767 | 1.94 | 144 | 90 |
| Wanuskewin Rd | 8.21 | 632 | 308 | 0.39 | 128 | 324 |
| AVERAGE (SD) | 8.17 (0.08) | 417 (311) | 845 (254) | 1.13 (0.77) | 121 (26.4) | 225 (121) |

*Sampled from SPs. All other 2020 samples were collected from on-site outlets or meltwater puddles.

**Sampled from municipal SW outfalls discharging effluent into the South Saskatchewan River. See Figure S1 for the delineation of relevant urban catchments.

Table S8. Chloride and sulphate analysis for select 2020 snow pile and snowmelt samples.

| Date | Sampling Site | Chloride (mg/L) | Sulphate (mg/L) |
| --- | --- | --- | --- |
| April 2nd 2019 | USask Campus (SM) | 150 | 8.2 |
| March 7th 2020 | Valley Rd (SP) | 380 | 21 |
|  | Valley Rd (SM) | 1,900 | 99 |
|  | USask Campus (SP) | 520 | 14 |
|  | USask Campus (SM) | 320 | 120 |
| March 26th 2020 | Valley Rd | 5,800 | 270 |
| March 29th 2020 | Valley Rd | 770 | 42 |
| April 17th 2020 | Valley Rd | 2,600 | 220 |
|  | Central Ave | 1,700 | 140 |
|  | Light & Power SW | 59 | 480 |
| April 23rd 2020 | Valley Rd | 2,900 | 240 |
|  | Central Ave | 1,600 | 120 |
| April 28th 2020 | Valley Rd | 810 | 57 |
|  | USask Campus | 110 | 8.9 |
|  | Central Ave | 1,100 | 110 |
| May 1st 2020 | Valley Rd | 1,000 | 89 |
| May 5th 2020 | Valley Rd | 380 | 50 |
|  | USask Campus | 290 | 61 |
| May 12th 2020 | Valley Rd | 410 | 46 |

Table S9. Metals species in 2019 snow samples. All samples were taken from the snow piles except one of the April 2/19 USask samples. Samples were prefiltered using 0.45 µm filters and prepared with 0.02M nitric acid. They were run using ICP-MS by the USask Geology labs. Refer to Table S2 for limits of detection/quantification.

|  | Sample Id | Al | As | Ba | Cd | Co | Cr | Cu 63 | Fe 56 | Cu 63 | Fe 56 | Mn | Mo | Ni | Pb | Sb | Se | Sn | Sr | Ti | Tl | U | V | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| April 2nd 2019 | USask (SP) | 120 | 0 | 50 | 0.52 | 0.47 | 0 | 135 | 72 | 135 | 72 | 103 | 4.87 | 6.65 | 2.41 | 0 | 16 | 0 | 100 | 1.94 | 0 | 0.67 | 1.76 | 108 |
|  | USask (SM) | 192 | 0 | 40 | 0.92 | 0.84 | 0 | 28 | 184 | 28 | 184 | 146 | 2.80 | 8.15 | 1.82 | 0 | 5 | 0 | 38 | 4.29 | 0 | 0.25 | 2.51 | 277 |
| April 13th 2019 | Valley Rd. | 401 | 0 | 17 | 0.82 | 0 | 0 | 114 | 113 | 114 | 113 | 19 | 2.02 | 0 | 6.82 | 0 | 12 | 0 | 17 | 2.71 | 0 | 0.14 | 0.00 | 106 |
|  | USask | 238 | 6.6 | 31 | 0.57 | 0.77 | 0 | 50 | 204 | 50 | 204 | 100 | 2.82 | 0 | 4.04 | 0 | 6 | 0 | 32 | 6.37 | 0 | 0.21 | 2.93 | 189 |
|  | Wanuskewin Rd. | 183 | 0 | 7 | 0 | 0 | 0 | 26 | 116 | 26 | 116 | 11 | 1.60 | 0 | 1.73 | 0 | 0 | 0 | 12 | 3.27 | 0 | 0.00 | 2.17 | 100 |
| April 18th 2019 | Valley Rd. | 107 | 0 | 16 | 0 | 0 | 0 | 24 | 86 | 24 | 86 | 18 | 1.59 | 0 | 4.14 | 0 | 0 | 0 | 13 | 2.17 | 0 | 0.00 | 1.77 | 136 |
|  | Wanuskewin Rd. | 119 | 26 | 13 | 0 | 0 | 0 | 11 | 85 | 11 | 85 | 19 | 1.62 | 0 | 0.76 | 0 | 0 | 0 | 14 | 2.56 | 0 | 0.00 | 0.00 | 95 |
|  | Central Ave. | 115 | 0 | 28 | 0 | 0.41 | 0 | 11 | 89 | 11 | 89 | 69 | 1.60 | 0 | 1.24 | 0 | 0 | 0 | 15 | 1.92 | 0 | 0.09 | 0.00 | 147 |
| April 24th 2019 | Valley Rd. | 82 | 0 | 20 | 0 | 0.38 | 0 | 10 | 49 | 10 | 49 | 27 | 4.38 | 0 | 3.54 | 0 | 0 | 0 | 14 | 0.00 | 0 | 0.16 | 0.00 | 110 |
|  | USask | 160 | 0 | 34 | 1.12 | 0.62 | 0 | 56 | 278 | 56 | 278 | 69 | 7.42 | 0 | 7.50 | 0 | 6 | 1 | 42 | 2.72 | 0 | 0.52 | 0.00 | 166 |
|  | Wanuskewin Rd. | 58 | 0 | 25 | 0 | 0.32 | 0 | 7 | 61 | 7 | 61 | 52 | 2.83 | 0 | 0.78 | 0 | 0 | 0 | 21 | 0.00 | 0 | 0.00 | 2.38 | 119 |
|  | Central Ave. | 111 | 0 | 63 | 0 | 1.16 | 0 | 7 | 92 | 7 | 92 | 195 | 2.86 | 0 | 0.94 | 0 | 0 | 0 | 39 | 0.00 | 0 | 0.00 | 0.00 | 128 |

Table S10. Metals species in 2020 snowmelt samples. All samples were taken from snowmelt pools (or the outlet at the Valley Rd. site) except one of the April 2/19 USask samples. Samples were prefiltered using 0.45 µm filters and prepared with 0.02M nitric acid. They were run using ICP-MS by the USask Geology labs. Refer to Table S2 for limits of detection/quantification.

|  | Element | Al | As | Ba | Cd | Co | Cr | 63 Cu | 65 Cu | 54 Fe | 56 Fe | Hg | Mn | Mo | Ni | Pb | Sb | Se | Sn | Sr | Ti | Tl | U | V | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mar 7th 2020 | Valley Rd. (SP) | 111 | 0.96 | 62 | 0.18 | 0.32 | 4.37 | 11 | 12 | 51 | 83 | 0.03 | 21 | 3.94 | 16.48 | 1.37 | 0.57 | 0.15 | 1.14 | 98 | 0.82 | 0.01 | 0.68 | 1.63 | 69 |
|  | Valley Rd. (SM) | 52 | 1.02 | 88 | 0.2 | 2.11 | 2.4 | 10 | 11 | 21 | 31 | 0.07 | 466 | 6.36 | 15.29 | 0.56 | 1.06 | 0.86 | 0.45 | 156 | 0.5 | 0.05 | 1.02 | 2.76 | 86 |
|  | USask (SP) | 41 | 1.31 | 77 | 0.13 | 0.14 | 1.27 | 13 | 14 | 18 | 29 | 0.02 | 8 | 2.08 | 6.45 | 0.53 | 0.48 | 0.55 | 2.5 | 242 | 0.75 | 0.01 | 2.2 | 1 | 56 |
|  | USask (SM) | 77 | 0.88 | 118 | 0.29 | 0.94 | 2.63 | 53 | 56 | 41 | 68 | 0.06 | 396 | 1.71 | 12.11 | 1.01 | 0.35 | 0.18 | 8.13 | 186 | 0.78 | 0.01 | 0.89 | 1.7 | 90 |
| Mar 26th 2020 | Valley Rd. | 28 | 1.85 | 177 | 0.29 | 2.14 | 2.15 | 42 | 26 | 27 | 34 | 0.05 | 574 | 9.8 | 21.14 | 0.47 | 1.19 | 2.23 | 1.03 | 430 | 0.64 | 0.1 | 2.21 | 3.88 | 68 |
| Mar 29th 2020 | Valley Rd. | 69 | 0.67 | 53 | 0.13 | 0.37 | 4.24 | 8 | 8 | 26 | 39 | 0.04 | 10 | 2.25 | 8.9 | 0.75 | 0.53 | 0.8 | 0.56 | 99 | 0.59 | 0.02 | 0.61 | 0.91 | 36 |
| April 10th 2020 | Valley Rd. | 146 | 0.73 | 60 | 0.1 | 0.32 | 3.71 | 26 | 29 | 116 | 182 | 0.09 | 18 | 2.38 | 6.16 | 2.82 | 0.52 | 0.53 | 2.6 | 104 | 1.83 | 0.01 | 0.82 | 1.57 | 66 |
| April 17th 2020 | Valley Rd. | 141 | 2.36 | 101 | 0.38 | 1.34 | 2.24 | 22 | 24 | 136 | 197 | 0.06 | 882 | 5.98 | 16.92 | 1.81 | 0.73 | 3.08 | 1.27 | 353 | 1.48 | 0.12 | 1.73 | 3.25 | 89 |
|  | USask | 30 | 1.08 | 87 | 0.2 | 0.14 | 1.2 | 9 | 10 | 32 | 54 | 0.02 | 10 | 2.12 | 7.83 | 0.73 | 0.31 | 0.24 | 2.48 | 230 | 0.73 | <0.02 | 1.11 | 0.99 | 88 |
|  | Wanuskewin Rd. | 84 | 0.62 | 44 | 0.07 | 0.13 | 2.67 | 7 | 8 | 27 | 39 | 0.02 | 7 | 2.55 | 5.47 | 0.54 | 0.51 | 0.18 | 1.05 | 86 | 0.63 | 0.01 | 0.47 | 0.56 | 21 |
|  | Central Ave. | 27 | 0.84 | 81 | 0.13 | 0.4 | 3.01 | 7 | 8 | 21 | 32 | 0.02 | 26 | 3.54 | 4.82 | 0.84 | 0.51 | 0.91 | 0.41 | 348 | 0.54 | 0.04 | 1.9 | 1.55 | 120 |
|  | Circle Dr. Bridge SE (SM) | 37 | 1.47 | 48 | 0.17 | 0.27 | 1.21 | 5 | 6 | 15 | 22 | 0.01 | 7 | 1.17 | 7.8 | 0.47 | 0.28 | 0.62 | 0.3 | 565 | 0.34 | <0.02 | 11.58 | 0.54 | 63 |
|  | 23rd St. E (SM) | 55 | 1.39 | 47 | 0.1 | 0.15 | 1.32 | 12 | 12 | 28 | 49 | 0.01 | 6 | 1.89 | 6.23 | 0.43 | 0.69 | 0.38 | 1.1 | 168 | 1.83 | <0.02 | 0.96 | 1.28 | 49 |
| April 20th 2020 | Vallley Rd. | 35 | 0.76 | 49 | 0.06 | 0.29 | 16.88 | 11 | 12 | 43 | 61 | 0.02 | 196 | 3.62 | 6.75 | 4.12 | 0.61 | 0.75 | 1.11 | 124 | 0.46 | 0.01 | 0.81 | 1.21 | 581 |
| April 23rd 2020 | Valley Rd. | 155 | 1.91 | 78 | 0.34 | 1.6 | 3.33 | 17 | 16 | 45 | 67 | 0.15 | 919 | 4.37 | 21.5 | 0.49 | 0.75 | 1.74 | 0.22 | 257 | 1.13 | 0.11 | 2.04 | 2.69 | 57 |
|  | USask | 104 | 0.81 | 57 | 0.05 | 0.21 | 0.86 | 8 | 8 | 63 | 103 | 0.02 | 18 | 1.5 | 2.54 | 0.32 | 0.42 | 0.27 | 0.11 | 114 | 3.11 | 0.01 | 0.59 | 0.88 | 13 |
|  | Wanuskewin Rd. | 98 | 0.75 | 48 | 0.03 | 0.16 | 1.22 | 5 | 6 | 35 | 57 | 0.02 | 12 | 2.07 | 3.69 | 0.28 | 0.46 | 0.13 | 0.04 | 74 | 1.79 | <0.02 | 0.53 | 0.6 | 20 |
|  | Central Ave. | 67 | 1.18 | 135 | 0.07 | 0.48 | 1.08 | 14 | 16 | 37 | 57 | 0.07 | 13 | 3.06 | 6.07 | 0.39 | 0.51 | 0.66 | 0.07 | 276 | 1.23 | 0.02 | 2.37 | 1.54 | 29 |
| April 28th 2020 | Valley Rd. | 102 | 1.04 | 61 | 0.07 | 0.39 | 1.14 | 10 | 11 | 69 | 113 | 0.03 | 23 | 2.36 | 5.48 | 0.35 | 0.6 | 0.55 | 0.12 | 108 | 2.93 | 0.03 | 0.86 | 1.53 | 17 |
|  | USask | 158 | 1.48 | 48 | 0.04 | 0.28 | 1.21 | 18 | 18 | 121 | 204 | 0.04 | 17 | 1.06 | 3.29 | 0.71 | 0.42 | 0.1 | 0.1 | 59 | 4.28 | 0.01 | 0.52 | 0.77 | 27 |
|  | Central Ave. | 86 | 1.31 | 84 | 0.05 | 0.48 | 1.42 | 13 | 14 | 49 | 81 | 0.05 | 9 | 4.46 | 4.56 | 0.32 | 0.71 | 0.68 | 0.06 | 224 | 1.73 | 0.03 | 2.38 | 1.77 | 17 |
| May 1st 2020 | Valley Rd. | 29 | 0.68 | 71 | 0.12 | 0.37 | 1.1 |  | 7 | 14 | 21 | 0.02 | 165 | 2.81 | 8.85 | 0.33 | 0.65 | 0.62 | 0.33 | 128 | 0.34 | 0.03 | 0.83 | 1.53 | 30 |
| May 5/20 | Valley Rd. | 48 | 0.49 | 46 | 0.1 | 0.12 | 1.07 | 4 | 5 | 20 | 33 | 0.01 | 7 | 0.99 | 6.74 | 0.61 | 0.46 | 0.07 | 0.17 | 52 | 0.59 | 0.01 | 0.23 | 0.46 | 52 |
|  | USask | 87 | 0.6 | 60 | 0.3 | 0.26 | 3.28 | 10 | 10 | 36 | 58 | 0.08 | 10 | 2.49 | 12.79 | 0.84 | 0.69 | 0.25 | 1.02 | 82 | 0.94 | 0.01 | 0.58 | 1.15 | 124 |
| May 12/20 | Central Ave. | 64 | 0.69 | 57 | 0.14 | 0.28 | 3.74 | 8 | 9 | 34 | 54 | 0.18 | 6 | 4.34 | 13.81 | 0.82 | 0.69 | 0.32 | 0.37 | 126 | 0.46 | 0.02 | 1.33 | 0.91 | 64 |
|  | Valley Rd. | 93 | 0.67 | 55 | 0.06 | 0.26 | 1.47 | 7 | 7 | 45 | 75 | 0.08 | 11 | 3.29 | 4.57 | 0.29 | 0.67 | 0.24 | 0.06 | 80 | 1.63 | 0.02 | 0.7 | 1.01 | 21 |
|  | USask | 105 | 0.7 | 47 | 0.05 | 0.15 | 1.11 | 6 | 6 | 40 | 69 | 0.05 | 26 | 1.64 | 3.91 | 0.34 | 0.42 | 0.11 | 0.06 | 67 | 1.93 | <0.02 | 0.36 | 0.64 | 17 |
|  | Wanuskewin Rd. | 180 | 1.3 | 68 | 0.14 | 0.34 | 2.44 | 19 | 20 | 89 | 151 | 0.06 | 16 | 2.24 | 6.09 | 0.83 | 0.59 | 0.18 | 0.11 | 79 | 1.64 | 0.01 | 0.71 | 0.7 | 49 |

Table S11. Dissolved PAHs (ng/L) detected in snow samples.

| Date | Sampling Site | Acenaphthylene | Fluorene | BbF | BkF | BaP | Benz[a]anthracene | Chrysene | Dibenz[a,h]anthracene | Pyrene | Phenanthrene | Anthracene | Indeno[123]cd-pyrene | Benzo[ghi]perylene | Sum PAHs (ug/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| April 2nd 2019 | USask Campus (SP) | 0.95 | 3.69 | 0 | 0 | 0.77 | 0 | 0 | 0 | 2.34 | 9.55 | 9.56 | 0.23 | 0.13 | 27.21 |
|  | USask Campus (SM) | 0.00 | 3.03 | 0 | 0 | 0.27 | 0 | 0 | 0.25 | 0.96 | 2.64 | 2.65 | 10.03 | 0.13 | 19.96 |
| April 13th 2019 | Valley Rd | 1.69 | 34.57 | 0.22 | 0.16 | 0.53 | 0.24 | 0.48 | 0.13 | 1.48 | 0.75 | 0.84 | 0.19 | 0.20 | 41.48 |
|  | USask Campus | 3.08 | 2.83 | 0.15 | 0.15 | 0.28 | 0.13 | 0.10 | 0 | 2.60 | 6.43 | 6.44 | 0.05 | 0.04 | 22.28 |
|  | Wanuskewin Rd | 0.71 | 22.12 | 0.00 | 0.00 | 1.00 | 0 | 0 | 0 | 71.7 | 317 | 182 | 0.06 | 0.08 | 595.10 |
| April 18th 2019 | Valley Rd | 1.35 | 2.44 | 0.16 | 0.13 | 1.30 | 0.76 | 0.77 | 0 | 2.66 | 6.24 | 6.25 | 0.24 | 0.18 | 22.50 |
|  | Wanueskewin Rd | 4.58 | 5.89 | 0 | 0 | 0.83 | 0 | 0 | 1.36 | 3.26 | 12.21 | 12.23 | 40.7 | 0.10 | 81.14 |
|  | Central Ave | 3.52 | 7.13 | 0 | 0 | 0.40 | 0 | 0 | 0 | 1.29 | 4.02 | 4.03 | 0.21 | 0.04 | 20.63 |
| April 24th 2019 | Valley Rd | 0.07 | 0.27 | 0.05 | 0.04 | 0.05 | 0.04 | 0.04 | 0 | 0.09 | 0.31 | 0.06 | 0.03 | 0.03 | 1.07 |
|  | USask Campus | 3.13 | 2.08 | 0 | 0 | 0 | 0 | 0 | 0 | 3.94 | 5.60 | 5.61 | 0.10 | 0.28 | 20.75 |
|  | Wanuskewin Rd | 2.52 | 4.53 | 0.33 | 0.33 | 0.27 | 0.55 | 0.56 | 0 | 5.60 | 13.0 | 13.0 | 0.16 | 0.10 | 40.96 |
|  | Central Ave | 4.67 | 3.60 | 0.31 | 0.31 | 0.40 | 0.42 | 0.42 | 0.10 | 7.62 | 11.6 | 1.26 | 0.20 | 0.13 | 31.06 |
| March 7th 2020 | Valley Rd. (SP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Valley Rd. (SM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | USask Campus (SP) | 0.10 | 0.14 | 0.04 | 0.04 | 0.05 | 0.03 | 0.02 | 0.02 | 0.05 | 0.01 | 0.03 | 0.04 | 0.03 | 0.59 |
|  | USask Campus (SM) | 0.09 | 0.07 | 0.03 | 0.03 | 0 | 0.01 | 0.01 | 0 | 0.04 | 0.03 | 0.02 | 0.02 | 0.01 | 0.36 |
| March 26th 2020 | Valley Rd | 0 | 0 | 0 | 0 | 0 | 0.03 | 0.03 | 0 | 0.04 | 0.03 | 0.04 | 0.02 | 0.01 | 0.20 |
| March 29th 2020 | Valley Rd | 0 | 1.00 | 0 | 0 | 0 | 0.01 | 0.01 | 0 | 0.04 | 0.02 | 0.02 | 0.02 | 0.01 | 1.14 |
| April 10th 2020 | Valley Rd | 0 | 0 | 0 | 0 | 0.03 | 0.02 | 0.02 | 0.01 | 0.03 | 0.02 | 0.03 | 0.02 | 0.01 | 0.18 |
| April 17th 2020 | Valley Rd | 0.04 | 0.03 | 0 | 0 | 0.03 | 0.02 | 0.02 | 0 | 0.03 | 0.01 | 0.02 | 0.01 | 0.00 | 0.20 |
|  | USask Campus | 0 | 0 | 0 | 0 | 0.12 | 0.01 | 0.02 | 0 | 0.03 | 0.01 | 0.03 | 0.02 | 0.01 | 0.24 |
|  | Wanuskewin Rd | 0 | 0 | 0 | 0 | 0.02 | 0.01 | 0.02 | 0.01 | 0.03 | 0.01 | 0.02 | 0.02 | 0.01 | 0.15 |
|  | Central Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Light & Power SW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23rd St E SW | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.03 | 0.03 | 0.04 | 0.02 | 0.02 | 0.14 |
| April 20th 2020 | Valley Rd | 0 | 0.15 | 0.00 | 0.00 | 0.00 | 0.01 | 0.02 | 0.01 | 0.03 | 0.02 | 0.02 | 0.02 | 0.01 | 0.30 |
|  | USask Campus | 0 | 0 | 0 | 0 | 4.64 | 0 | 0 | 0 | 0.05 | 0.05 | 0.06 | 0.02 | 0.01 | 4.82 |
|  | Wanuskewin Rd | 0.39 | 0 | 0 | 0 | 0 | 0 | 0 | 0.04 | 0.03 | 0.01 | 0.02 | 0.04 | 0.04 | 0.56 |
|  | Central Ave | 0 | 0 | 0.12 | 0.12 | 0.22 | 0 | 0 | 0.00 | 0.10 | 0.11 | 0.12 | 0.01 | 0.02 | 0.83 |
| April 28th 2020 | Valley Rd | 0.12 | 0 | 0.04 | 0.04 | 0.04 | 0.02 | 0.02 | 0.02 | 0.04 | 0.01 | 0.02 | 0.03 | 0.03 | 0.42 |
|  | USask Campus | 0.21 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.09 | 0.16 | 0.46 |
|  | Central Ave | 0 | 0 | 0.12 | 0.12 | 0.13 | 0.07 | 0.10 | 0.06 | 0.05 | 0.03 | 0.04 | 0.10 | 0.09 | 0.90 |
| May 1st 2020 | Valley Rd | 0.05 | 0 | 0.02 | 0.02 | 0.00 | 0.01 | 0.02 | 0.01 | 0.03 | 0.01 | 0.02 | 0.02 | 0.01 | 0.22 |
| May 5th 2020 | USask Campus | 0.26 | 0 | 0 | 0 | 0 | 0 | 0 | 0.01 | 0.04 | 0.03 | 0.02 | 0.02 | 0.01 | 0.39 |
|  | Central Ave | 0 | 0 | 0 | 0 | 0.03 | 0.01 | 0.02 | 0 | 0.03 | 0.02 | 0.02 | 0.02 | 0.01 | 0.16 |
| May 12th 2020 | Valley Rd | 0 | 0.07 | 0 | 0 | 0 | 0.02 | 0.02 | 0 | 0 | 0.03 | 0.04 | 0 | 0.00 | 0.18 |
|  | USask Campus | 0 | 0 | 0.04 | 0.03 | 0.05 | 0.01 | 0.02 | 0 | 0.03 | 0.02 | 0.04 | 0.02 | 0.01 | 0.27 |
|  | Wanuskewin Rd | 0 | 0 | 0 | 0 | 0.04 | 0 | 0 | 0 | 0.05 | 0.05 | 0.06 | 0.01 | 0.01 | 0.21 |
|  | CCME Guidelines |  | 3.0 |  |  | 0.015 | 0.018 |  |  | 0.025 | 0.4 | 0.012 |  |  |  |

##### Figure S1. Stormwater catchment map for the CoS. Each catchment comprises a mix of land uses (see Table S4). The road surface area from each catchment was used to estimate the area of impervious traffic surfaces, which was used to calculate the volume of snow transported to storage facilities.