Online Resource 2

HarvestField: A Decision-Support Framework for Robot-Oriented Kiwifruit Canopy Management Using Spatial Harvestability Modelling

Journal of Biosystems Engineering

Caption. Experimental protocol and parameter settings used for HarvestField simulation, calibration, optimisation, ablation, and statistical analysis.

# 1. Study unit and workflow

The experimental unit was one representative New Zealand-style kiwifruit bay at harvest time. Each candidate management vector was translated into phenologically ordered actions, simulated to harvest time using the FSPM, evaluated by spatial harvestability fields, and scored by HYI, yield proxy, and labour-cost proxy. All stochastic experiments were repeated with independent random seeds where applicable.

# 2. Management decision variables

| Variable | Symbol | Reference value | Bounds | Operation |
| --- | --- | --- | --- | --- |
| Cane spacing | sc | 0.30 m | 0.20–0.60 m | Winter pruning |
| Retained fruiting canes per bay side | nc | 15 | 8–20 | Winter pruning |
| Bud retention density | ρb | 20 buds m-1 | 10–65 buds m-1 | Bud thinning |
| Target fruit count per fruiting shoot | nf | 6 | 3–10 | Fruit thinning |
| Maximum node count after summer tipping | nt | 8 | 5–18 | Summer tipping |
| Cane training angle | θc | 90 degrees | 45–135 degrees | Cane training |

# 3. Canopy simulation protocol

Canopy development was represented by an L-system-based functional–structural plant model. The model state included geometric support structures, leaves, branches, fruit objects, and peduncles. Seasonal operations were applied in the following order: winter pruning, bud thinning, fruit thinning, summer tipping, and cane training. The simulated harvest-time geometry was stored and then passed to the field-computation module.

# 4. Field-computation settings

| Setting | Symbol | Value |
| --- | --- | --- |
| Bay length | Lb | 5.0 m |
| T-bar crossbar width | Wb | 1.5–1.8 m |
| Voxel resolution | Δ | 50 mm |
| Voxel grid size | — | 100 x 36 x 42 = 151,200 voxels |
| Reachability samples | Nq | 50,000 joint configurations |
| Visibility viewpoints | Nv | 8 viewpoints |
| Bootstrap replications | B | 1,000 |
| Baseline replications | R | 10 |

# 5. Harvestability field evaluation

Reachability was computed from sampled collision-free robot configurations and normalised manipulability. Visibility was computed by ray casting from candidate camera viewpoints. Clearance was computed as an exponential function of local obstacle density. Detachability combined the gripper-force term with a nearest-neighbour fruit-spacing term. The composite field was evaluated as

ℋ(x)=R(x)V(x)C(x)D(x).

# 6. Calibration procedure

The clearance parameters λ and rc were calibrated using exhaustive grid search. The target value was the published robotic kiwifruit harvesting success rate of 0.558. The accepted calibration point in the manuscript was λ=1.0 m−1 and rc=0.133 m, yielding HYI = 0.562.

# 7. Optimisation settings

| Parameter | Value |
| --- | --- |
| Algorithm | NSGA-III |
| Objectives | Maximise HYI, maximise normalised yield proxy, minimise normalised labour cost |
| Population size | 92 |
| Generations | 200 |
| Total evaluations per optimisation | 18,400 |
| Reference directions | Das-Dennis simplex-lattice, H = 12, M = 3 |
| Crossover | Simulated binary crossover, eta_c = 30 |
| Mutation | Polynomial mutation, eta_m = 20 |
| Implementation | Python 3.10; L-Py v3.14; OpenAlea/Caribu; pymoo v0.6.1 |

# 8. Baseline experiments

| Code | Strategy | Purpose |
| --- | --- | --- |
| B1 | Standard management | Commercial-reference management baseline |
| B2 | Yield-only optimisation | Tests whether yield maximisation aligns with harvestability |
| B3 | Random search | Controls for evaluation-budget effects |
| B4 | Single-objective HYI optimisation | Isolates the effect of the multi-objective formulation |
| B5 | Uniform field assumption | Tests the value of spatial heterogeneity |
| B6 | HarvestField-optimised | Full framework evaluation |

# 9. Ablation and controlled thinning experiments

A1 removed visibility by evaluating ℋ=RCD. A2 removed clearance by evaluating ℋ=RVD. A3 replaced the Caribu radiation model with a Beer-Lambert approximation. A4 compared ℋ-guided fruit thinning against uniform thinning under equal fruit counts. In A4, canopy realisation, robot configuration, random seed, field resolution, FSPM parameters, light model, and total fruit count were controlled within each paired replicate.

# 10. Sobol sensitivity analysis

The global sensitivity analysis varied six parameters: λ, rc, Fg, fruit set rate, budbreak rate, and individual leaf area. With base sample size N=512 and p=6 parameters, the number of model evaluations was

NSA=N(2p+2)=512(2×6+2)=7,168.

First-order, total-order, and second-order indices were computed. Parameters with ST>0.10 were treated as influential.

# 11. Statistical analysis

Pairwise comparisons were analysed with the Wilcoxon signed-rank test because the stochastic sample size was small and normality was not assumed. Baseline comparisons used a Bonferroni-adjusted threshold of α∗=0.05/8=0.00625. Effect size was reported using Cliff’s delta.

# 12. Output files retained for reproducibility

The retained outputs included JSON summaries, calibration grids, bootstrap HYI samples, optimisation archives, Sobol raw arrays, statistical-test outputs, field visualisation arrays, and selected generated figures. These files are organised in Online Resource 4 and Online Resource 5.