Table S1. MRI protocol parameters for tumor assessment

| Parameters | Repetition time (ms) | Echo time (ms) | Thickness (mm) | Intersection gap (mm) | Matrix (frequency × phase) | Band width (kHz) | Numbers of excitation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T2WI | 5200 | 76 | 3.5 | 0.5 | 320×320 | 42 | 1.5 |
| DWI | 2200 | Minimum | 3.5 | 1.0 | 128×160 | 250 | b value- dependent* |
| DCE (50 phase) | 3.8 | Minimum | 4.0 | 0 | 192×160 | 142.86 | 1 |

*: b values of DWI sequences: 0, 10, 25, 50, 80, 100, 150, 200, 500, 800, 1000, 1500, 2000; the numbers of excitation: b value-dependent (1,1,1,1,1,1,2,2,4,8,10,12,16).

Table S2. 5-fold cross-validation of habitat analysis

| Accuracy | Kappa | mtry | Resample |
| --- | --- | --- | --- |
| 0.75 | 0.18181818 | 2 | Fold1 |
| 0.85294118 | 0.47852761 | 2 | Fold2 |
| 0.74285714 | 0.0974212 | 2 | Fold3 |
| 0.74285714 | 0.16 | 2 | Fold4 |
| 0.8 | 0.41247002 | 2 | Fold5 |

Figure S1. Habitat clustering results. (a) Calinski–Harabasz score line chart and (b) scatter plot of clustering analysis on IVIM, DKI, and ADC mapping

Figure S2. (a) Bar chart of the mean SHAP value displays the order of the importance of the features. (b) Beeswarm plot of SHAP value demonstrates the impact of features on model output.