Supplementary Information

Electron generation in water induced by magnetic effect and its impact on dissolved oxygen concentration

Augustine Chung Wei Yap1, Hwang Sheng Lee1*, Joo Ling Loo2 and                             Nuruol Syuhadaa Mohd3

1Department of Mechanical and Material Engineering, Lee Kong Chian Faculty of Engineering and Science, Universiti Tunku Abdul Rahman, Sungai Long Campus, Jalan Sungai Long, Bandar Sungai Long, Cheras 43000, Kajang, Selangor, Malaysia.

2Department of Mechatronics and Biomedical Engineering, Lee Kong Chian Faculty of Engineering and Science, Universiti Tunku Abdul Rahman, Sungai Long Campus, Jalan Sungai Long, Bandar Sungai Long, Cheras 43000, Kajang, Selangor, Malaysia.

3Department of Civil Engineering, Faculty of Engineering, University of Malaya, Lembah Pantai, 50603 Kuala Lumpur, Malaysia.

* Correspondence: leehs@utar.edu.my

Table S1 Average measured values of water parameters before and after magnetization under different magnet arrangements

| Non-reversed polarity magnet arrangements |  |  |  |
| --- | --- | --- | --- |
| Parameters | Before magnetization | After magnetization | Average delta value () |
| pH | 5.65 0.005 | 5.85 0.005 | 0.20 0.01 |
| ORP (mV) | 378 1 | 362 1 | -16 2 |
| DO (mg/L) | 7.30 0.008 | 7.99 0.000 | 0.69 0.008 |
| Reversed polarity magnet arrangements |  |  |  |
| pH | 5.65 0.01 | 5.79 0.01 | 0.14 0.02 |
| ORP (mV) | 366 0.5 | 355 0 | -11 0.5 |
| DO (mg/L) | 7.63 0.005 | 8.01 0.005 | 0.38 0.01 |

Table S2 Average measured values of water parameters before and after magnetization under different flow rates

| 0.1-0.5 mL/s |  |  |  |
| --- | --- | --- | --- |
| Parameters | Before magnetization | After magnetization | Average delta value () |
| pH | 5.41 0.01 | 5.60 0.02 | 0.19 0.02 |
| ORP (mV) | 392 2 | 365 3 | -27 5 |
| DO (mg/L) | 7.30 0.05 | 7.71 0.02 | 0.41 0.07 |
| 1.0-1.5 mL/s |  |  |  |
| pH | 5.48 0.01 | 5.62 0.01 | 0.14 0.02 |
| ORP (mV) | 386 0.5 | 379 0.5 | -7 1 |
| DO (mg/L) | 7.32 0.03 | 7.67 0.03 | 0.35 0.06 |
| 2.0-2.5 mL/s |  |  |  |
| pH | 5.46 0.004 | 5.51 0.008 | 0.05 0.01 |
| ORP (mV) | 387 2 | 382 1 | -5 3 |
| DO (mg/L) | 7.35 0.005 | 7.56 0.016 | 0.21 0.02 |

Table S3 Theoretical E values calculated from Nernst equation based on pH 0 to 7 and E = 0.623V

| pH | [H+] | log10[H+] | [OH-] | E | Theoretical E |
| --- | --- | --- | --- | --- | --- |
| 0.00 | 1.00 | 0 | 1.00 × 10-14 | 0.623 | 0.623 |
| 0.50 | 3.16 × 10-1 | -0.5 | 3.16 × 10-14 | 0.623 | 0.616 |
| 1.00 | 1.00 × 10-1 | -1 | 1.00 × 10-13 | 0.623 | 0.608 |
| 1.50 | 3.16 × 10-2 | -1.5 | 3.16 × 10-13 | 0.623 | 0.601 |
| 2.00 | 1.00 × 10-2 | -2 | 1.00 × 10-12 | 0.623 | 0.593 |
| 2.50 | 3.16 × 10-3 | -2.5 | 3.16 × 10-12 | 0.623 | 0.586 |
| 3.00 | 1.00 × 10-3 | -3 | 1.00 × 10-11 | 0.623 | 0.579 |
| 3.50 | 3.16 × 10-4 | -3.5 | 3.16 × 10-11 | 0.623 | 0.571 |
| 4.00 | 1.00 × 10-4 | -4 | 1.00 × 10-10 | 0.623 | 0.564 |
| 4.50 | 3.16 × 10-5 | -4.5 | 3.16 × 10-10 | 0.623 | 0.557 |
| 5.00 | 1.00 × 10-5 | -5 | 1.00 × 10-9 | 0.623 | 0.549 |
| 5.50 | 3.16 × 10-6 | -5.5 | 3.16 × 10-9 | 0.623 | 0.542 |
| 6.00 | 1.00 × 10-6 | -6 | 1.00 × 10-8 | 0.623 | 0.534 |
| 6.50 | 3.16 × 10-7 | -6.5 | 3.16 × 10-8 | 0.623 | 0.527 |
| 7.00 | 1.00 × 10-7 | -7 | 1.00 × 10-7 | 0.623 | 0.520 |

Fig. S1 Relationship between pH and E value (ORP) based on theoretical Nernst calculation. The () plot corresponds to the correlation between E value and pH value while () plot corresponds to the correlation between OH ions concentration and pH value. The equation and R2 value for both plots are shown.

Table S4 Sample calculation of electron generated in water with magnetic effect based on pH and ORP

| Magnetization experiment (Non-reversed polarity and 0.1-0.5 mL/s flow rate) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | pH | pOH a | [OH-] (mol/L) b | Number of moles of OH- c | Number of moles of electron d | Number of electron e |
| Initial | 5.41 | 8.59 | 2.57 × 10-9 | 2.57 × 10-10 | 2.57 × 10-10 | 1.55 × 1014 |
| After flowing | 5.60 | 8.40 | 3.98 × 10-9 | 3.98 × 10-10 | 3.98 × 10-10 | 2.40 × 1014 |
| Control experiment |  |  |  |  |  |  |
| Initial | 5.40 | 8.60 | 2.51 × 10-9 | 2.51 × 10-10 | 2.51 × 10-10 | 1.51 × 1014 |
| After flowing | 5.44 | 8.56 | 2.75 × 10-9 | 2.75 × 10-10 | 2.75 × 10-10 | 1.66 × 1014 |

a pOH = 14 – pH

b [OH-] = Concentration of OH- = 10 -pOH

c Number of moles of OH-in 100 mL water=OH-mol 1 L×1 L1000 mL×100 mL

d Number of moles of electron = Number of moles of OH- (since mole ratio of OH- to electron = 1:1 according to reaction in Equation (S1))

4OH-⇋2H2O+O2+4e-	(S1)

e Number of electron = Number of moles of electron × NA where NA represents Avogadro’s number (6.02 × 1023)