SUPPLIMENTARY MATERIAL

Supplementary Table S1: The protocol for staining, dehydration and embedding (EPON) for sample preparation of the coral fragment using the Pelco Biowave to view with SEM and confocal autofluorescence microscopy. Dehydration and infiltration steps can be added/removed as needed depending on size and permeability of the sample. Osmium step was removed for confocal autofluorescence microscopy.

| Processing Phase |  | Microwave Setting (Watts) |  | Time of Procedure |  | Sample under Vacuum |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Buffer Rinse |  |  |  |  |  |  |  |
| 0.1 M Sodium Cacodylate |  | N/A |  | Fumehood;10 minutes |  | No |  |
| 0.1 M Sodium Cacodylate |  | 80 |  | 40 s |  | No |  |
| Osmium tetroxide fix |  |  |  |  |  |  |  |
| 2 x Osmium + Ruthinium Red |  | 80 |  | 2 min on, 2 min off, 2 on |  | Yes |  |
| Water Rinse |  |  |  |  |  |  |  |
| 2 x Water |  | 80 |  | 40 s |  | No |  |
| Dehydration (Ethanol) |  |  |  |  |  |  |  |
| 3 x 20% (steps repeated with 10% increments) |  | 250 |  | 40 s |  | No |  |
| 3x 70% (left overnight) |  | 250 |  | 40 s |  | No |  |
| 3x 80% |  | 250 |  | 40 s |  | No |  |
| 3x 90% |  | 250 |  | 40 s |  | No |  |
| 3 x 100% (left over night) |  | 250 |  | 40 s |  | No |  |
| Dehydration (Acetone) |  |  |  |  |  |  |  |
| 3 x 100% (Acetone) (left overnight) |  | 250 |  | 40 s |  | No |  |
| Resin infiltration (Spurrs) |  |  |  |  |  |  |  |
| 15% |  | 250 |  | 3 min |  | Yes |  |
| 30% |  | 250 |  | 3 min |  | Yes |  |
| 50% |  | 250 |  | 3 min |  | Yes |  |
| 70% (left overnight) |  | 250 |  | 3 min |  | Yes |  |
| 90% |  | 250 |  | 3 min |  | Yes |  |
| 3 x 100% (left overnight) |  | 250 |  | 3 min |  | Yes |  |

Supplementary Movie S1 (.Mp4 format). Peak mesenterial filament behaviour during over 4 days of the coral’s immune response as seen through a glass substrate (Fig. S2). Ciliates and other colonisers populated the mucus biofilm that developed in this time.

Supplementary Movie S2 (.Mp4 format). The mesenterial filaments twisting to move through temporary holes in the coral fragments soft tissues to gain access to the substrate outside of the coral fragments body (Fig. S3).

Supplementary Movie S3 (.Mp4 format). Enlarged tissues at the contact points between the coral fragment and a ceramic substrate developing and anchoring onto the substrate (Fig. S2).

Supplementary Movie S4 (.Mp4 format). The mesenterial filaments removed the coral's tissues through autolysis (Fig. S6).

Supplementary Movie S5 (.Mp4 format). Localised pulsing of the corals tissues to move the lappet appendage along the surface depositing the corals initial skeleton (Fig. S7).