Supplementary Information

Domain multiplexed computer-generated holography by embedded wavevector filtering algorithm

Lin Wu1,2 and Ziyang Zhang1,2*

1Key Laboratory of 3D Micro/Nano Fabrication and Characterization of Zhejiang Province, School of Engineering, Westlake University, 18 Shilongshan Road, Hangzhou 310024, Zhejiang Province, China.

2Institute of Advanced Technology, Westlake Institute for Advanced Study, 18 Shilongshan Road, Hangzhou 310024, Zhejiang Province, China.

*zhangziyang@westlake.edu.cn

1. Spatial frequency filtering by space-limited phase distribution in SLM plane

The GS algorithm embedded with spatial frequency filtering can generate images with significantly improved quality. Our strategy to imbed the spatial frequency filtering is to modulate the input or output phases of the forward transform equation, or even the forward transform equation itself. Most current modified GS algorithms modulate the output phases or amplitudes of the forward transform equation, while few works modulate the input phases [1,2]. The interpretations of these methods are not systematically analyzed and claimed. Here, we provide a unified interpretation of the three proposed methods as spatial frequency filtering. In the first three sections of this Supplementary Information, we verify our methods both theoretically by equation derivations and experimentally. Hence, the functions of the spatial frequency filtering can be confirmed.

Figure S1. The GS algorithm embedded with spatial frequency filtering using the windowed Fourier transform and the inverse Fourier transform.

The first method we employed is the standard GS algorithm incorporated with windows for constraining phase distribution in SLM plane, as depicted in Figure S1. The conventional GS algorithm processes the un-modified phase distributions in the SLM plane directly in the next iteration. The proposed algorithm, however, multiplies the phase distributions with a window before performing the Fourier transform. This kind of Fourier transform is termed as windowed Fourier transform, and has been demonstrated useful in phase retrieval for fringe patterns [3, 4]. As far as we know, the combination of a windowed Fourier transform and an inverse Fourier transform has not been implemented in any CGH algorithm.

The first two steps in the modified GS algorithm are the windowed Fourier transform and the inverse Fourier transform, which can be represented as:

(1)

(2)

By inserting Eq. 1 into Eq. 2, we have:

(3)

Here, we define:

(4)

From Eq. 3 and 4, we can obtain:

(5)

Therefore,  is from the convolution of the exponential of  with the inverse Fourier transform of a real valued two-dimensional matrix. Figure S2 shows an example for this calculation. The resulted amplitude distributions in Figure S2c vary “fast” over the region covering the nonzero phases in Figure S2a and remain almost flat outside that region. This is consistent with the results in Figure S1, where the region that  varies “fast” is the same to that of non-zero .

The results denote that a window constraint on the phase distribution can affect the amplitude distribution after a Fourier transform and an inverse Fourier transform. As the amplitudes in the center region of  are much larger than the surrounding regions, the phases in the center region of  play a more important role in generating the final image . This phenomenon is also discussed using a space-limited version of a phase exponential function for demonstrating that the local spatial frequencies can localize to certain spatial regions in the real space [5]. However, the local spatial frequency has not been applied for achieving spatial frequency filtering so far, to the best of our knowledge.

Figure S2. The calculations of the convolution of the exponential of a space-limited phase distribution with the inverse Fourier transform of a random real valued two-dimensional matrix.

After the iterations converge, the resulted image can be obtained as:

(6)

From Figure S1, the inverse Fourier transform of Eq. 6 can also be presented as:

(7)

From Eq. 5 and 7, we have:

(8)

The same results can also be obtained from Eq. 3. The calculated results of Eq. 8 for different window sizes are depicted in Figure S3. The results denote that the space-limited random phase distributions can filter spatial frequencies. The size of the passband of the filter is determined by the window size.

The experimental results obtained by using different window sizes are depicted in Figure S4. In Figure S4, the dimensions of the whole matrices are 480480, and the sizes of the non-zero regions are 480480, 240240 and 4040 in Figure S4a, S4c and S4e, respectively. Figure S4d denotes that by filtering appropriate amount of spatial frequencies, the quality of images can be significantly improved. When the passband of the filter is narrow, the image generated by the CGH algorithm is shown in Figure S4f.

Figure S3. The calculated results for the spatial frequency filtering by using windows for the phase distribution in the SLM plane.

Figure S4. The measured results for the spatial frequency filtering by using different windows for phase distribution in SLM plane.

2. Spatial frequency filtering by “slow” varying phase distribution in image plane

We also propose another scheme to filter spatial frequencies within the GS algorithm. Form Eq. 1-5, the window size of  not only decides the size of the region where  varies “fast”, but also decides whether the variations of  are "slow" or “fast” in the (u, v) plane. Therefore, besides using the windowed Fourier transform, one can also modify the phase distribution  to achieve spatial frequency filtering. The relationship between  and  can be obtained by inserting Eq. 2 into Eq. 6:

(9)

The above equation can be rewritten as:

(10)

Therefore, by only changing the gradients of , one can also filter spatial frequencies in the target image. Figure S5 depicts the calculated results from Eq. 10 for different random . The sizes of the super unit cells in Figure S5a, S5d, S5g and S5j are 33, 66, 110 and 101, respectively. The results show that the size of the region where  is not flat is determined by the gradients of . By comparing Figure S5c and S5f, the “slower” the variation of  is, the more components of the target image are filtered. Form Figure S5i and S5l, this scheme can also filter particular spatial frequencies in the image by changing the gradients of  along specific directions.

The experimental results generated by the GS algorithm embedded with the different kinds of modifications on  are depicted in Figure S6. In Figure S6b and S6c, the equation used for  is:

(11)

where s is an integer within [−1, 1] and [−2, 2] for Figure S6b and S6c, respectively. The results show that by filtering some spatial frequencies, the generated hologram can be significantly improved. However, the quality of the image will deteriorate when too much spatial frequencies are filtered out. The filtering of particular spatial frequencies is also realized experimentally, as shown in Figure S6d and S6e. The above analyses denote that the spatial frequency filtering can result in images deviated from the target images, and the spatial frequency filtering can be implemented within the GS algorithm by only modifying either the phase distribution in the SLM plane  or that in the image plane .

Figure S5. The calculated results for the spatial frequency filtering by using “slow” varying phase distributions in image plane.

The Eq. 8 and 10 confirm two schemes to filter out the spatial frequencies from the target image  by multiplexing a window function to  and smoothing the variation of , respectively. Both methods do not directly modulate the , so its size is kept identical throughout the iterations and can cover the full region of the SLM. In contrast, the electrical implementation of the conventional spatial frequency filter placed in the pupil plane of a 4-f system cannot be employed in the iterative algorithm, as it will change the size of .

Figure S6. The measured results from CGH generated by the GS algorithm embedded with the spatial frequency filtering realized by “slow” varying phase distributions in image plane.

3. Spatial frequency filtering by modifying the forward diffraction equation

Apart from modifying the input or output phases of the forward transmission equation, it is also possible to modify the forward transmission equation itself to realize spatial frequency filtering. The distance between two points in the SLM plane and image plane used for generating the images in Figure S7 is:

(12)

In Figure S7a, a = b = 1, the speckle is presence in the generated letter. By employing the low-pass filter by setting a = 1 and b = 6, the image smooth can be accomplished, as depicted in Figure S7b. Other wavevector filters can be also be realized, such as the bandpass filter depicted in Figure S7c by setting a = 6 and b = 1.

Figure S7. The measured results generated by the GS algorithms with (a) un-modified forward transmission equation, (b) modified forward transmission equation as low-pass wavevector filter, (c) modified forward transmission equation as band-pass wavevector filter.

4. Holographic multiplexing algorithm

The flow chart of the proposed holographic multiplexing algorithm is depicted in Figure S8. The Fourier transform and the RS diffraction equation are applied to images to be projected with and without a lens. The operations highlighted by the red box correspond to the spatial frequency filtering. The algorithm can integrate with other holographic multiplexing methods to achieve multiple image projections.

Figure S8. Flow chart of the proposed holographic multiplexing algorithm with spatial frequency filtering.

References

- Wyrowski F, Bryngdahl O. Speckle-free reconstruction in digital holography. J Opt Soc Am A 1989; 6: 1171.

- Bräuer R, Wyrowski F, Bryngdahl O. Diffusers in digital holography. J Opt Soc Am A 1991; 8: 572.

- Kemao Q. Two-dimensional windowed Fourier transform for fringe pattern analysis: Principles, applications and implementations. Opt Lasers Eng 2007; 45: 304–317.

- Kemao Q, Wang H, Gao W. Windowed Fourier transform for fringe pattern analysis: theoretical analyses. Appl Opt 2008; 47: 5408.

- J. W. Goodman, Introduction to Fourier Optics, 4th Ed., Ch. 2, Section 2.2, W. H. Freeman, 2017.