Table A1: Emissions factors from 1 kg of e-waste part

| SimaPro 8.3.0.0 | kg CO2/kg |
| --- | --- |
| PS recycling of PS ¦ Alloc Def, S | 0.016 |
| Steel and iron recycling of steel and iron ¦ Alloc Def, S | -0.015 |
| Recycling aluminium/RER S demo7 | -10.850 |
| treatment of shredder fraction from manual dismantling, shredding GLO | 0.010 |
| treatment of used Li-ion battery, pyrometallurgical treatment GLO | 0.051 |
| treatment of used Li-ion battery, hydrometallurgical treatment GLO | 0.035 |
| treatment of waste electric and electronic equipment, shredding GLO | 0.003 |
| treatment of used desktop computer, manual dismantling RoW | 0.006 |
| treatment of electronics scrap, metals recovery in copper smelter RoW | 0.005 |
| treatment of precious metal from electronics scrap, in anode slime, precious met | 0.042 |

Table A2:  GWP

| Greenhouse Gas | Formula | 100-year GWP (AR4) |
| --- | --- | --- |
| Carbon dioxide | CO2 | 1 |
| Methane | CH4 | 25 |
| Nitrous oxide | N2O | 298 |
| Sulphur hexafluoride | SF6 | 22,800 |
| Hydrofluorocarbon-23 | CHF3 | 14,800 |
| Hydrofluorocarbon-32 | CH2F2 | 675 |
| Perfluoromethane | CF4 | 7,390 |
| Perfluoroethane | C2F6 | 12,200 |
| Perfluoropropane | C3F8 | 8,830 |
| Perfluorobutane | C4F10 | 8,860 |
| Perfluorocyclobutane | c-C4F8 | 10,300 |
| Perfluoropentane | C5F12 | 13,300 |
| Perfluorohexane | C6F14 | 9,300 |

Source :Climate Change Connection (2020)

Table A3: Flow of discard television (km)

| From household to Kalasin dismantling communities (km) | Recycling industries | From dismantling community to recycling industries (km) |
| --- | --- | --- |
| 31.08-193.4 | Copper | 441.4-549.0 |
|  | Steel | 112.5-521.5 |
|  | Plastic | 31.1-518.5 |
|  | Circuit | 523.4 |
|  | Aluminum | 545.4 |
| Transportation distance to Buriram dismantling communities (km) |  |  |
| 97.4 - 448.7 | Copper | 441.4-549.0 |
|  | Steel | 65.1-433.5 |
|  | Plastic | 441.3-467.3 |
|  | Circuit | 142.8-392.1 |
|  | Aluminum | 419.1 |

Table A4: Flow of discard air conditioner (km)

| Transportation distance to Sua Yai Uthit dismantling communities (km) | Recycling industries | From dismantling community to recycling industries (km) |
| --- | --- | --- |
| 11.7-72.1 | Copper | 14.1-88.4 |
|  | Aluminum | 88.4-171.4 |
|  | Circuit | 30.9 |
|  | Plastic | 30.9-88.4 |

Table A5: Flow of discard computer (km)

| Transportation distance to Kalasin dismantling communities (km) | Recycling industries | From dismantling community to recycling industries (km) |
| --- | --- | --- |
| 31.08-193.4 | Copper | 441.4-549.0 |
|  | Steel | 112.5-521.5 |
|  | Plastic | 31.1-518.5 |
|  | Circuit | 523.4 |
|  | Aluminum | 545.4 |
| Transportation distance to Buriram dismantling communities (km) |  | From dismantling community to recycling industries (km) |
| 97.4 – 448.7 | Copper | 441.4-549.0 |
|  | Steel | 65.1-433.5 |
|  | Plastic | 441.3-467.3 |
|  | Circuit | 142.8-392.1 |
|  | Aluminum | 419.1 |
| Transportation distance to Sua Yai Uthit dismantling communities (km) |  | From dismantling community to recycling industries (km) |
| na | Copper | 53.4 |
|  | Steel | 53.4 |
|  | Plastic | 30.9 |
|  | Circuit | 63.2 |
|  | Aluminum | 53.4 |

Table A6: Flow of discard cell phone

|  | To Mahasarakham dismantling community (km) |  | To recycling industries (km) | km |
| --- | --- | --- | --- | --- |
| Nationwide | 32.3-1,610.9 |  | Circuit and battery | 505.7 |

Table A7: Summary of emission factors

|  | Petroleum | Global warming potential | Life cycle inventory databases |
| --- | --- | --- | --- |
| Transportation | 4-wheet truck 0% loading (1.5 tons Diesel) | 0.2400 Kg CO2 eq/ton-km | Thai National LCI database/MTEC |
|  | 4-wheet truck 100% loading (1.5 tons Diesel) | 0.2140 Kg CO2 eq/ton-km | Thai National LCI database/MTEC |
|  | 6-wheel truck 100% loading (8.5 tons Diesel) | 0.0649 Kg CO2 eq/ton-km | Thai National LCI database/MTEC |
|  | 6-wheel truck 0% loading (8.5 tons Diesel) | 0.4943 Kg CO2 eq/ton-km | Thai National LCI database/MTEC |
| Electricity | Electricity | 0.6093 Kg CO2 eq/Kwh | Thailand Grid mix 2009 |

Table A8: Purchasing price of parts from discarded television

|  | Source | THB/kg |
| --- | --- | --- |
| Plastic | Secondhand shops and dismantling communities | 1-4 |
| Steel | Secondhand shops and dismantling communities | 9.5-9.9 |
| Copper | Secondhand shops and dismantling communities | 170-195 |
| Aluminum | Secondhand shops and dismantling communities | 25-38 |
| Circuit (Copper extraction) | Secondhand shops and dismantling communities | 3-5 |
| Total |  |  |

Table A9: Selling price of computer parts

| Part | Source | THB/kg |
| --- | --- | --- |
| Plastic | Secondhand shops and dismantling communities | 2-3 |
| Steel | Secondhand shops and dismantling communities | 5-9.9 |
| Copper | Secondhand shops and dismantling communities | 150-195 |
| Aluminum | Secondhand shops and dismantling communities | 25-38 |
| Circuit | Secondhand shops and dismantling communities | 20-80 |
| Silver | The London Metal Exchange* | 24,378.3 |
| Gold | The London Metal Exchange | 1,773,273.0 |
| Palladium | The London Metal Exchange | 2,442,321.9 |
| Lead | The London Metal Exchange | 60.1 |

Table A10: Selling price of air conditioner

| Part | Source | THB/Kg |
| --- | --- | --- |
| Plastic | Secondhand shops and dismantling communities | 1-8.5 |
| Steel | Secondhand shops and dismantling communities | 7-9.9 |
| Copper | Secondhand shops and dismantling communities | 156-170 |
| Aluminum | Secondhand shops and dismantling communities | 30-38 |
| Circuit | Secondhand shops and dismantling communities | 10-15 |

Table A11: Selling price of cellphone

| Part | Source | THB/Kg |
| --- | --- | --- |
| Circuit | Secondhand shops and dismantling communities | 600-800 |
| Battery | Secondhand shops and dismantling communities | 35 |

Table A12: Transportation costs

| (One way from A to B) |  |  |  |
| --- | --- | --- | --- |
| Pickup |  | 0.96 | THB/ton-km |
| 6-wheels |  | 0.63 | THB/ton-km |

Source: Department of Transportation (2020)

References

Climate Change Connection. (2020) CO2 equivalents. Available at: https://climatechangeconnection.org/emissions/co2-equivalents/.

Department of Transportation. (2020) Truck data service center: Transportation costs. Available at: https://www.thaitruckcenter.com/tdsc/index.

Gaidajis G, Angelakoglou K and Aktsoglou D. (2010) E-waste: environmental problems and current management. Journal of Engineering Science and Technology Review 3: 193-199.