METHODS

Sample processing for metabolomics – Metabolomic processing of serum samples was conducted at University of Iowa Core Laboratory. To extract metabolites, serum samples were diluted in 18 volumes of ice-cold 2:2:1 methanol/acetonitrile/water containing a mixture of internal standards (D4-citric acid, D4-succinic acid, D8-valine, and U13C-labeled glutamine, glutamic acid, lysine, methionine, serine, and tryptophan; Cambridge Isotope Laboratories), where the 1-part water was composed of sample volume + water. An additional deuterated short chain fatty acid standard was also added for signal normalization. Serum extraction mixtures were vortexed for 10 minutes at RT and rotated for 1 hour at -20°C. Mixtures were centrifuged for 10 minutes at 21,000 x g, and 150 µL of the cleared metabolite extracts were transferred to autosampler vials and dried using a SpeedVac vacuum concentrator (ThermoFisher, CA, USA). Dried metabolite extracts were reconstituted in 30 μL of 11.4 mg/mL methoxyamine (MOX) in anhydrous pyridine, vortexed for 5 minutes, and heated for 1 hour at 60°C. Next, to each sample 20 μL of N,O-Bis(trimethylsilyl)trifluoroacetamide (TMS) was added, samples were vortexed for 1 minute, and heated for 30 minutes at 60°C.

Derivatized samples were analyzed by gas chromatography-mass spectrophotometry (GC-MS) with 1 μL of derivatized sample was injected into a Trace 1300 GC (ThermoFisher, CA, USA) fitted with a TraceGold TG-5SilMS column (ThermoFisher, CA, USA) operating under the following conditions: split ratio = 20:1, split flow = 24 μL/minute, purge flow = 5 mL/minute, carrier mode = Constant Flow, and carrier flow rate = 1.2 mL/minute. The GC oven temperature gradient was as follows: 80°C for 3 minutes, increasing at a rate of 20°C/minute to 280°C, and holding at a temperature at 280°C for 8 minutes. Ion detection was performed by an ISQ 7000 mass spectrometer (ThermoFisher, CA, USA) operated from 3.90 to 21.00 minutes in EI mode (-70eV) using select-ion monitoring (SIM).

Raw data were processed using TraceFinder 5.1 (ThermoFisher, CA, USA). Metabolite identification and annotation required at least two ions (target + confirming) and a unique retention time that corresponded to the ions and retention time of a reference standard. A pooled sample generated prior to derivatization was analyzed at the beginning, at a set interval during, and the end the analytical run to correct peak intensities using the NOREVA tool [1]. NOREVA corrected data were then normalized to the total signal per sample to control for extraction, derivatization, and/or loading effects.

Additional Analysis and Supplementary Data Visualization - Data processing, statistical analysis, and visualization were performed using R Statistical Software (version 3.3.1). Raw and MetaboAnalyst-adjusted datasets were imported and wrangled using the readxl and tidyverse (version 2.0.0) suite of packages, primarily relying on dplyr and tidyr for data transformation. For the clinical gastrointestinal (GI) symptom analysis, longitudinal changes between baseline (T1) and following first or second cycle of chemotherapy (T2) were evaluated. The normality of paired differences was first assessed using the Shapiro-Wilk test, dictating the use of either a parametric paired t-test or a non-parametric Wilcoxon signed-rank test. Relationships between GI symptom severity and distress were evaluated using the Spearman rank correlation coefficient. All data visualizations were generated using the ggplot2 package.

REFRENCES

1.	Li B, Tang J, Yang Q, Li S, Cui X, Li Y, et al. NOREVA: normalization and evaluation of MS-based metabolomics data. Nucleic Acids Res. 2017;45(W1):W162-w70.