Appendix:

Table A1. Some of the most important studies conducted on microplastics produced from tire wear

| Reference | Year | Operational Factor |  |  |  | Tire Type | Analysis Method | Key Finding Qualitative |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Traffic & Driving | Vehicle & Tire | Environmental & Weather | Mitigation & Management |  |  |  |
| (Kole et al., 2017) | 2017 | * |  |  |  | Mixed Fleet | Systematic Review, Modeling & Simulation | Global Significance: Major source of microplastics (5-10% ocean plastics, 3-7% PM2.5). |
| (Turner, 2018) | 2018 |  |  |  | * | Mixed Fleet | XRF spectrometry, GC-MS, ICP-MS, FTIR | TWPs spread globally, carrying toxic additives (Zn, PAHs, heavy metals) that pose environmental and health risks. |
| (Sommer et al., 2018) | 2018 | * |  |  |  | Mixed Fleet | Microscopy & Imaging | Particle Properties: TWPs are large, composite particles; Zinc is not a reliable tracer. |
| (Hüffer et al., 2019) | 2019 |  | * |  |  | Unspecified/ General Fleet | Systematic Review, Spectroscopy & Chromatography | Particle Properties: Complex composite with dual sorption mechanisms; carbon black role is key. |
| (Unice et al., 2019) | 2019 | * |  |  |  | Mixed Fleet | Modeling & Simulation | Environmental Fate: Only 2% of TRWP exported to estuary; 90% trapped in freshwater sediments. |
| (Leads and Weinstein, 2019) | 2019 | * |  |  |  | Unspecified/ General Fleet | Microscopy & Imaging, Spectroscopy & Chromatography | Environmental Abundance: TWP was 2nd most abundant MP type; higher near high-traffic areas. |
| (Järlskog et al., 2020) | 2020 |  | * |  |  | Unspecified/ General Fleet | Physicochemical Methods, Microscopy & Imaging | Environmental Abundance & Management: TBMP dominant on streets; sweeping collects significant mass. |
| (Parker et al., 2020) | 2020 |  |  | * |  | Unspecified/ General Fleet | Microscopy & Imaging, Spectroscopy & Chromatography | Biological Effects: First evidence of TWP ingestion in wild fish. |
| (Tamis et al., 2021) | 2021 |  |  |  | * | Passenger Vehicles | Modeling & Simulation | Ecological Risk: TWP & associated chemicals pose risk (PEC/PNEC>1) to aquatic life. |
| (Wang et al., 2021) | 2021 | * |  |  |  | Unspecified/ General Fleet | Systematic Review | Global Significance: Major source in all environmental compartments; transport varies. |
| (Luo et al., 2021) | 2021 |  |  |  | * | Unspecified/ General Fleet | Systematic Review | Mitigation: Highlights RTC and TRD as understudied sources; discusses solutions. |
| (Kukkola et al., 2021) | 2021 |  | * |  |  | Unspecified/ General Fleet | Literature Review | Freshwater Significance: TWPs are a major but understudied freshwater microplastic source, with key gaps in fate and combined toxicity research. |
| (Prenner et al., 2021) | 2021 | * | * | * | * | Mixed Fleet, buses) | Static Material Flow Analysis, Emission Factor modeling, XRF, particle size distribution analysis | Austria 2018: TWP emissions: 2.4 kg/cap, 6% air, 94% soil/water. Non‑exhaust emissions: 9% of national TSP. 2050: Shift to EVs raises TWP unless mileage is cut. |

Table A1. Some of the most important studies conducted on microplastics produced from tire wear (Continued)

| Reference | Year | Operational Factor |  |  |  | Tire Type | Analysis Method | Key Finding Qualitative |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Traffic & Driving | Vehicle & Tire | Environmental & Weather | Mitigation & Management |  |  |  |
| (Navarro et al., 2022) | 2022 |  |  | * |  | Mixed Fleet | Biological Assays | Biological Effects: No adverse effects on invertebrates at environmentally relevant doses. |
| (Budhiraja et al., 2022) | 2022 |  |  |  | * | Passenger Vehicles | Physicochemical Methods | Analytical Method: Developed magnetic extraction method for TWP; weathering reduces recovery. |
| (Corella-Puertas et al., 2022) | 2022 |  |  | * |  | Heavy-Duty/Commercial Vehicles | Systematic Review, Modeling & Simulation | Environmental Fate: TRWP are negatively buoyant, sediment quickly; low characterization factors. |
| (Sun et al., 2022) | 2022 |  | * |  |  | Mixed Fleet | Spectroscopy & Chromatography, Physicochemical Methods | Toxicity & Regionality: Higher TRWMP in N. cities; NR more cytotoxic than SBR/BR; induces oxidative stress. |
| (Li et al., 2022) | 2022 |  |  | * |  | Unspecified/ General Fleet | Biological Assays | Biological Effects: Inhaled TWMPs induce pulmonary fibrotic injury in mice via specific mechanism. |
| (Järlskog et al., 2022) | 2022 | * | * | * |  | Mixed Fleet | Microscopy & Imaging | Environmental Abundance & Fate: TBWP significant in road dust; finer fraction dominates; runoff has highest proportion. |
| (Rødland et al., 2022) | 2022 | * |  | * | * | Mixed Fleet | Spectroscopy & Chromatography, Modeling & Simulation | Environmental Fate & Management: Strong TSS-TRP relationship; gully pots retain TRWP; treatment 63% efficient. |
| (Weinstein et al., 2023) | 2023 | * |  |  |  | Passenger Vehicles | Spectroscopy & Chromatography, Microscopy & Imaging | Environmental Abundance: Major, underreported component of MP pollution in urban estuaries. |
| (Brožová et al., 2023) | 2023 |  |  | * |  | Passenger-car tire | SEM, FT-IR, XPS, elemental analysis, particle size distribution, dispersion stability tests | Reference Materials: Cryo-milling produces aged TWPs (~5 μm) with rapid fragmentation and stable dispersion, suitable for ecotoxicity test standards. |
| (Goßmann et al., 2023) | 2023 |  |  | * |  | Unspecified/ General Fleet | Spectroscopy & Chromatography | Environmental Fate: TWP detected in remote marine atmosphere; suggests long-range transport. |
| (Shin et al., 2023) | 2023 |  | * |  |  | Unspecified/ General Fleet | Modeling & Simulation | Environmental Fate & Risk: Most TWPs deposit in rural soil (78%), threat to soil ecosystems. |
| (Liu et al., 2023) | 2023 | * |  |  | * | Passenger Vehicles | Spectroscopy & Chromatography | Ecological Risk & Management: Carcinogenic risk exceeded safety threshold; road cleaning reduces levels. |
| (Wang et al., 2023) | 2023 |  |  |  | * | Unspecified/ General Fleet | Systematic Review | Research Trends: Research hotspots: biotoxicity, distribution, health risks; meta-analysis shows variable TWP content. |

Table A1. Some of the most important studies conducted on microplastics produced from tire wear (Continued)

| Author | Year | Operational Factor |  |  |  | Tire Type | Analysis Method | Key Finding Qualitative |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Traffic & Driving | Vehicle & Tire | Environmental & Weather | Mitigation & Management |  |  |  |
| (Klun et al., 2023) | 2023 |  |  | * |  | Unspecified/ General Fleet | Physicochemical Methods, Spectroscopy & Chromatography | Environmental Fate: TWM not biodegradable; aging changes density but not composition (persistent). |
| (Wang et al., 2023) | 2023 |  | * |  |  | Unspecified/ General Fleet | Modeling & Simulation | Mitigation: Adjusting additive ratios can reduce TWP toxicity to aquatic organisms. |
| (Ly and El-Sayegh, 2023) | 2023 | * |  |  |  | Mixed Fleet | Modeling & Simulation | Review Synthesis: TRWP are a major source; accurate emission prediction requires integrated models. |
| (Xiao et al., 2024) | 2024 | * |  | * |  | Passenger Vehicles | Modeling & Simulation | Environmental Fate: River retention drastically reduces TWP ocean flux; changes dominant ocean sources. |
| (Wang et al., 2024) | 2024 |  |  | * |  | Unspecified/ General Fleet | Biological Assays | Biological Effects: Toxicity of TMPs to algae is concentration-dependent and exacerbated by warming. |
| (Hoeke et al., 2024) | 2024 | * |  | * | * | Mixed Fleet | Modeling & Simulation | Management: Estimates national emissions and environmental pathways; user phase & artificial turf are major sources. |
| (Kim et al., 2024) | 2024 |  |  |  | * | Unspecified/ General Fleet | Physicochemical Methods, Spectroscopy & Chromatography | Mitigation: Fe-impregnated biochar showed high adsorption capacity for MPs. |
| (Knight et al., 2024) | 2024 | * | * |  |  | Passenger Vehicles | Systematic Review, Modeling & Simulation | Mitigation: EV-specific design and tires can mitigate wear; mass alone overestimates BEV wear. |
| (Halsband et al., 2024) | 2024 |  | * |  |  | Unspecified/ General Fleet | Systematic Review | Review Synthesis: complex composition, additives, analytical gaps, leaching, uptake, risk assessment needs. |
| (Wang et al., 2024) | 2024 | * |  |  |  | Mixed Fleet | Spectroscopy & Chromatography | Ecological Risk: Combined pollution and risk of TRWMPs and heavy metals (e.g., Cr) is a major concern. |
| (Foetisch et al., 2024) | 2024 | * |  | * | * | Mixed Fleet | Microscopy & Imaging | Analytical Method & Management: Developed ML method for TWP in soil; concentration decreases with distance from road. |
| (Giechaskiel et al., 2024) | 2024 | * |  |  |  | Unspecified/ General Fleet | Modeling & Simulation | Review Synthesis: Aggregated average wear rate data from numerous studies. |
| (Mierzynska et al., 2024) | 2024 | * |  | * |  | Unspecified/ General Fleet | Microscopy & Imaging, Spectroscopy & Chromatography | Environmental Abundance & Fate: MP/TWP concentration increases with traffic intensity; snow is a temporary reservoir. |

Table A1. Some of the most important studies conducted on microplastics produced from tire wear (Continued)

| Author | Year | Operational Factor |  |  |  | Tire Type | Analysis Method | Key Finding Qualitative |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Traffic & Driving | Vehicle & Tire | Environmental & Weather | Mitigation & Management |  |  |  |
| (Chebbi et al., 2024) | 2024 |  |  | * |  | Unspecified/ General Fleet | Physicochemical Methods, Biological Assays | Biological Effects: MPs in polluted soils increase metal uptake in plants, reduce growth, cause oxidative stress. |
| (Özen and Mutuk, 2025) | 2025 | * |  | * |  | Mixed Fleet | Spectroscopy & Chromatography, Microscopy & Imaging | Environmental Abundance: Black fragment MPs dominated; correlated with PM10 and traffic flow. |
| (Sommer et al., 2025) | 2025 | * | * |  |  | Unspecified/ General Fleet | Physicochemical Methods | Wear Rates & Significance: Measured bike tire wear rates; rear > front; contributes <1% to national MP emissions. |
| (Zou et al., 2025) | 2025 |  |  | * |  | Mixed Fleet | Bibliometric & Thematic Analysis | Soil Pollution: TWPs are toxic and understudied in soils, releasing persistent harmful chemicals with underestimated ecological risks. |
| (Beaurepaire et al., 2025) | 2025 |  |  | * |  | Unspecified/ General Fleet | Spectroscopy & Chromatography | Environmental Abundance: TRWP concentration in soil orders of magnitude higher than other MPs. |
| (Piarulli et al., 2025) | 2025 |  | * |  |  | Unspecified/ General Fleet | Spectroscopy & Chromatography | Biological Effects: Leached chemicals, not particles, are the primary driver of toxicity in cod. |
| (Keang et al., 2025) | 2025 |  |  | * |  | Mixed Fleet | FE-SEM, ICP-MS, Acute toxicity test, Biochemical assays, Transcriptomics (RNA-Seq) | Ecotoxicological Effects: Mixtures of tire particles and microplastics induce stronger oxidative stress in Daphnia magna than individual pollutants. |
| (Yaya et al., 2025) | 2025 |  |  |  | * | Unspecified/ General Fleet | Physicochemical Methods | Mitigation: Optimal MP substitution in cement tailings increases strength. |
| (Premarathna et al., 2025) | 2025 | * | * | * |  | Mixed Fleet | Spectroscopy & Chromatography, Microscopy & Imaging | Environmental Abundance: Roadside MPs dominated by TWP; abundance depends on traffic/road conditions. |
| (Otieno et al., 2025) | 2025 |  |  | * | * | Unspecified/ General Fleet | Spectroscopy & Chromatography, Biological Assays | Biological Effects & Risk: TMPs altered microbial community and increased antibiotic resistance genes in wastewater. |
| (Müller et al., 2025) | 2025 |  |  |  | * | Unspecified/ General Fleet | Systematic Review | Ecological Risk: TWP particles show low risk; leachates (e.g., 6PPD-Q) pose higher risk. |