Sustainably Producing Green Fuel Ammonia in Equatorial Waters and Piping It Worldwide, and Removing Atmospheric CO2 to Return to the 1960s Temperature Rise Level by 2100

Hiroshi Kobayashi

Professor Emeritus of Tokyo Denki University, 5 Senju-Asahi-cho, Adachi-ku, Tokyo, Japan; 
hirokoba@mail.dendai.ac.jp; Tel.: +81-3-3353-9889

Supplementary Information

Supplementary Note:

Kobayashi, H. Carbon-free energy supply system using floating offshore photovoltaics and hydrogen energy carriers: Towards the achievement of the Paris agreement goal. J. Jpn Soc. Energy Resour. 43, 274–284 (2022).

Permission to share this article was obtained from the Japan Society of Energy and Resources.

The English-translated version of the PDF file of the aforementioned paper (Reference 7) is avalable at https://www.cck.dendai.ac.jp/s/xpnox67e8rdapquinx94mgzh44sub4ew/JSER-H_Kobayashi-.pdf.

Supplementary Fig. S1. Floating offshore photovoltaic (FOPV) plant featuring photovoltaic (PV) panels connected flexibly in a honeycomb structure [7].

Supplementary Fig. S2. Four-layer connection circuit of the proposed floating offshore photovoltaic (FOPV) plant, designed to prevent potential-induced degradation (PID) in photovoltaic (PV) modules [14]. By connecting fine aggregates consisting of 10 PV panels in series and then connecting them in parallel, the maximum potential difference is limited to ±250 V. With this configuration, the FOPV system as a whole has a four-layer structure.

Supplementary Fig. S3. Properties of carbon steel pipes used for high-pressure piping [22]. (a) shows the design pressure and service pressure based on the schedule (Sch); (b) shows the outer diameter (OD), inner diameter (ID), and wall thickness (WT) of Sch 160 pipes.

Supplementary Fig. S4. Temperature characteristics of liquefied ammonia (LNH3). The physical properties of NH3 were calculated using FLUIDAT [48]. At 20 °C and 0.1 MPa, NH3 exists as a gas but liquefies when pressurized at ≥0.86 MPa.

Supplementary Fig. S5. Performance analysis results of liquefied ammonia (LNH3) pipeline transportation. Stable LNH3 transport requires real-time control of numerous parameters.

Supplementary Fig. S6. Historical trends and projections of global warming from 1850 to 2100. (a) Shows the influence and use of fossil fuel energy (FFE). (b) Shows the influence and use of traditional clean energy (TCE) (renewable, nuclear, and hydropower). Although rapid growth is expected, by 2100, the TCE amount will only reach 29% of the FFE amount and account for 22% of the total primary energy. (c) Carbon dioxide (CO2) emission factor from FFE. Before 1970, when coal was the main source, the emission factor was high. Subparts (d) to (g) denote various GHG components. CO2 emissions from non-FFE are caused by land-use change and land management (e.g., deforestation, degradation, regeneration after farmland abandonment, and peat drainage). (h) This illustrates the SSP3-7.0 scenario, in which current global warming policies continue (resulting in cumulative CO2 emissions doubling by 2100). The surface temperature will rise to 3.6 ℃ [2.8–4.6 ℃] by 2100 [30,34,35]

Supplementary Fig. S7. Radiative forcing and temperature rise based on shared socioeconomic pathway (SSP) scenarios. This figure shows the relationship between radiative forcing and temperature rise under four scenarios. If current climate mitigation measures continue (Scenario 4), the temperature rise will reach 3.6 °C [2.8–4.8 °C] by 2100. In contrast, if CO2eq is removed from the atmosphere to return to 1960s temperature rise levels, the radiative forcing will decrease to a negative value, meaning the Earth will cool.

Supplementary Fig. S8. Photovoltaic (PV) panel spacing and light transmittance. As shown in Fig. S9b, the FOPV plant comprises 1 million hexagonal PV panels (length = 3 m per side) flexibly connected in a honeycomb structure. The PV panels are spaced 0.6 m apart, but this interval can be increased. Additionally, to avoid the effects of tsunamis and ocean currents, the system is installed at water depths ≥50 m and in areas with ocean currents ≤1 knot. This figure shows the PV panel spacing and light transmittance (assuming that 70 % of the space between PV panels is light-transmissive, accounting for light blockage by connectors), with light transmittance ranging from 0% to 45% in the spacing range of 0 m to 3 m. The impact of changes in light transmittance on ecosystems and environmental conditions varies significantly depending on the target time and space. For example, a light transmittance of 9% to 45% during the spring algal bloom period can have a significant impact in highly productive ocean areas. In less productive ocean areas during winter, the impact is likely to be minimal. To mitigate these impacts, it is necessary to conduct large-scale field trials to evaluate various environmental effects—such as impacts on marine primary production and the ecosystems that depend on it, changes in local ocean currents due to alterations in momentum flux at the sea surface, and changes in ocean stratification due to alterations in heat flux—and to explore mitigation measures. However, since FOPV plants occupy less than 1% of the area in equatorial waters, the overall impact on the ocean is limited.

Supplementary Table S1. Performance specifications of the floating offshore photovoltaic (FOPV) plant proposed in this study (see [7]). This table shows example performance specifications for FOPV based on an annual average global solar irradiance of 240 W/m2 in equatorial waters. By flexibly joining one million PV panels per unit to form a honeycomb structure, the system disperses and absorbs the complex impact forces exerted by waves and wind-driven waves. (see Fig. S1-2 and Table 1).

| Parameter | Value | Unit and remarks |
| --- | --- | --- |
| FOPV structure | Four-layer honeycomb structure |  |
| Length of a hexagon's side (Hpv) | 3 | m |
| PV panel area (Spv) | 23.4 | m2, = 3(√3/2Hpv2) |
| PV panel thickness (Hpv) | 60 | cm |
| Number of PV panels in fine aggregate (Nfp) | 10 | PV panels |
| Small aggregate capacity (Csa) | 10 | fine aggregates |
| Medium aggregate capacity (Cma) | 100 | small aggregates |
| Largr aggregate capacity (Cla) | 100 | medium aggregates |
| Number of PV panels in FOPV (Nfopv) | 1,000,000 | PV panels, = Nfp･Csa･Cma･Cla |
| Spacing between PV panels (Ppv) | 60 | cm |
| FOPV area (Sfopv) | 28.3 | km2, = 3(√3/2(Hpv+Ppv/2)2)･Nfopv |
| PV cell effective area of PV panel (Sepv) | 20.0 | m2 |
| Maximum all-sky solar radiation (Rmax) | 1,000 | W/m2 |
| Annual average all-sky solar radiation (Ravg) | 240 | W/m2 |
| PV module conversion efficiency (Epv) | 24.0 % |  |
| PV panel nominal maximum output (Wpv) | 4.80 | kW, = Sepv･Rmax･Epv |
| PV panel DC output voltage (Vpv) | 50.0 | V |
| PV panel maximum output current (Apv) | 96.0 | A, = Wpv/Vpv |
| Fine aggregate maximim output power (Wfa) | 48.0 | kW. = Wpv･Nfp |
| Fine aggregate maximum output voltage (Vfa) | ±250 | V, for PID measures |
| Fine aggregate maximum output current (Afa) | 96.0 | A, = Wfa/Vfa = Apv |
| Small aggregate maximum output power (Wsa) | 480 | kW, = Wfa･Csa |
| Small aggregate maximum output voltage (Vsa) | ±250 | V, = Vfa |
| Small aggregate maximum output current (Sma) | 960 | A, = Wsa/Vsa |
| DC-DC convertor efficence (Efs) | 96 % | see Fig. S5. |
| Medium aggregate maximum output power (Wma) | 46.1 | MW, = Wsa･Cma･Efs |
| Medium aggregate maximum output voltage (Vma) | ±250 | kV, = Vsa･Cma |
| Medium aggregate maximum output current (Ama) | 92.2 | A |
| Large aggregate maximum output power (Wla) | 4.61 | GW, = Wma･Cla |
| Large aggregate maximum output voltage (Vla) | ±250 | kV, = Vma |
| Large aggregate maximum output current (Ala) | 9.22 | kA, = Wla/Vla |
| FOPV operating rate (Rop) | 85 % | for maintenance |
| FOPV annual average power generation (Wfopv) | 940 | MW/plant, = Nfopv･Sepv･Ravg･Epv･Efs･Rop |
| FOPV annual average current (Ifopv) | 1.88 | kA, = Wfopv/Vla, Ala/Ifopv = 4.90 |
| FOPV annual power generation capacity (Pfopv) | 8.23 | TWh/plant, = Wfopv･(hours per year) |

Abbreviations: floating offshore photovoltaic (FOPV), photovoltaic (PV)

Table S2. Example of construction time for an FOPV plant using AI-equipped transport and assembly robots (Fig. S2, Table S1, Table 1). An FOPV consisting of 1 million PV panels has a four-layer structure and is constructed using AI-equipped transport and assembly robots designed for each layer. Assuming an assembly time of 300 hours per layer, the bottom layer targets 100,000 fine aggregates. Since each fine aggregate consists of 10 PV panels and assuming an assembly time of 20 minutes per aggregate, 111 robots are required. If robots are allocated for the upper layers using the same approach, the FOPV plant can be constructed in 88 days. This group of robots can construct 42 FOPVs over 10 years, and 433 such groups would be sufficient to construct CFE2050 (36,161 FOPVs) over 20 years. However, robots for large aggregates can be shared among multiple FOPV plants. Note that assembly of the upper layer begins once one-quarter of the lower layer’s construction is complete. The daily working hours are 10 hours, and assuming a 60% annual availability rate to account for inclement weather and other factors.

| Processes | Number of targets | Number of robots | Unit assembly time (min.) | Total assembly time |  |
| --- | --- | --- | --- | --- | --- |
| Assembly of fine aggregate | 100,000 | 111 | 20 | 300 | hours |
| Assembly of small aggregate | 1,000 | 3.3 | 60 | 300 | hours |
| Assembly of medium aggregate | 100 | 0.7 | 120 | 300 | hours |
| Assembly of large aggregate | 1 | 0.013 | 240 | 300 | hours |
| Construction days per FOPV plant |  |  |  | 88 | days |
| FOPV plants constructible within 10 years |  |  |  | 42 | FOPVs |
| Robot clusters required to achieve CFE2050 |  |  |  | 433 | clusters |

Supplementary Table S3. Comparison of hydrogen energy carrier (HEC) characteristics with existing energy sources [45,46]. The energy density of uranium is more than 10,000 times that of fossil fuels; however, the issues with the disposal of nuclear fuels have yet to be solved. In addition, methanol has attracted attention as a carbon-neutral synthetic fuel, as it can be synthesized from H2, and CO₂ obtained from power plants and the atmosphere. However, the combustion of synthetic fuels results in the emission of CO₂ into the atmosphere. As energy consumption increases (see Fig. 1a), if synthetic fuels are produced as a substitute for fossil fuels, 42 Gt and 60 Gt of CO₂ will require CDR by 2050 and 2100, respectively. Furthermore, to return the temperature rise to 1960s levels by 2100, these amounts must be added to the CDR volumes specified in Scenarios 1–3. The HECs listed in the table below offer a fundamental solution to these challenges. There are four types of HECs: high-pressure hydrogen gas (H2), liquid hydrogen (LH2), liquefied ammonia (LNH3), and methylcyclohexane (MCH). Among these, LNH3 has the highest volumetric and mass energy density, and as the leading candidate, research on its practical application as a fuel is currently underway worldwide.

| Hydrogen energy carriers | Volumetric energy density | Mass energy density | Storage conditions | Long-term storage | Remarks |
| --- | --- | --- | --- | --- | --- |
| High pressure hydrogen gas (H2) | △ 767/1,200 Wh/L | ◎ 32,900 Wh/kg | △ 35/70 MPa | △ | △ Highly flammable, explosive |
| Liquid hydrogen (LH2) | 〇 2,330 Wh/L | ◎ 32,900 Wh/kg | ✖ －253 ℃ | ✖ | △ Highly flammable, explosive |
| Fuel liquefied ammonia (LNH3) | 〇 3,960 Wh/L | 〇 5,810 Wh/kg | ◎ Room temp, 0.86 MPa | ◎ | △ Acute toxicity, 〇 Many achievements |
| Methylcyclohexane (MCH) | △ 1,550 Wh/L | △ 2,010 Wh/kg | ◎ Room temperature | 〇 | 〇 Flammability comparable to gasoline |
| (ref.) Liquefied natural gas (LNG) | ◎ 6,977 Wh/L | ◎ 15,167 Wh/kg | ✖ －161.5 ℃ | △ | ✖ CO2 emissions |
| (ref.) Crude oil | ◎ 10,628 Wh/L | ◎ 12,541 Wh/kg | ◎ Room temperature | ◎ | ✖✖ Large amount of CO2 emissions |
| (ref.) Coal | ◎ 9,972 Wh/L | 〇 7,150 Wh/kg | ◎ Room temperature | ◎ | ✖✖ Large amount of CO2 emissions |
| (ref.) Li-ion secondary battery | ✖ 520 Wh/L | ✖ 201 Wh/kg | ◎ Room temperature | ✖ | ✖ Lithium is a rare and expensive metal |
| (ref.) Uranium fuel | ◉ 7.6×109 Wh/L | ◉ 4.2×108 Wh/kg | 〇 Cooled in fuel pool | ✖Cooling required | ✖✖✖ Final disposal method not defined |

Note: The symbols ◉, ◎, 〇, △, ✖, ✖✖, and ✖✖✖ in table represent good to bad quality from left to right. Abbreviation: Hydrogen Energy Carrier (HEC).

Supplementary Table S4. Numerical analysis of liquefied ammonia (LNH3) pipeline transport performance. This table shows a numerical analysis of LNH3 pipeline transport characteristics using the Goal Seek function in Microsoft Excel. This method incorporates variable values from the analysis results, along with standard reference values.

| Parameter | Variable value | Unit | Reference value | Remarks |
| --- | --- | --- | --- | --- |
| Safety factor (SF) | 1.50 |  | 1.5 |  |
| Ambient temperature for pipe burial (T) | 35 | ℃ | 35 | See Fig. S7a |
| Dynamic viscosity of LNH3 (σ) | 1.29E-04 | Pa･s | 1.29E-04 | Pa･s = kg/(m･s), see Fig. S7b |
| Density of LNH3 (ρ) | 601 | kg/m3 | 601 | See Fig. S7c |
| Kinematic viscosity of LNH3 (ν) | 2.16E-07 | m2/s | 2.16E-07 | = σ/ρ |
| Speed of moving fluid (V) | 2.07 | m/s | 2.07 |  |
| Inner diameter of pipe (ID) | 0.532 | m | 0.532 | Sch160 |
| Reynolds number (Re) | 5.11E+06 |  | 5.11E+06 | = V･ID/ν |
| Cross section area of pipe (A) | 0.445 | m2 | 0.445 | = πID2/2 |
| Pipe inner surface roughness (ε) | 0.150 | mm | 0.150 | Steel pipe in use (uniform rust) condition |
| Coefficient of pipe friction (λ) | 0.0149 |  | 0.0149 | 1/√λ+2 log(ε/3.71*ID+2.51/Re√λ) = 0 |
| LNH3 flow rate (Q) | 0.921 | m3/s | 0.922 | = A･V |
| LNH3 flow rate per day (Qw) | 47,836 | tons/day | 47,879 | = ρQ*3600*24/1000 |
| Velocity head (Hv) | 0.219 | m | 0.219 | = V2/2g |
| Applied pressure (Pa) | 12.9 | MPa | 12.9 | = design pressure*2/3 |
| Pressure head (Hp) | 2,190 | m | 2,190 | = P/(ρg) |
| Pipe length between relay pumps (L) | 300 | km | 300 |  |
| Difference in elevation (ΔH) | 0 | m | 0 | ΔHp = ΔH･ρg |
| Pressure loss (ΔP) | 10.9 | MPa | 10.9 | = λL/d･ρV2/2 |
| NH3 liquid sustaining pressure (Pr) | 2.05 | MPa | 2.05 | Vapor pressure at T ℃ x 1.5, see Fig. S7a |
| Pressure pump efficiency (η) | 90 % |  | 90 % | Energy conversion efficiency without fevering |
| Pressure pump energy (Ep) | 13.2 | MW | 13.2 | = (Hv+Hp)･Qρg/η, here g = 9.8 m/s2 |
| Annual average power generation of FOPV (Wfopv) | 940 | W/plant | 940 | Average annual power generation of FOPV |
| Electricity-to-LNH3 conversion efficiency (Eas) | 74.0 % |  | 74.0 % | Including nitrogen air separation efficiency |
| Daily production of LNH3 by FOPV (Qpv) | 2,593 | tons/day | 2593 | = Wfopv･Eas･Esc･Edc/5,810 Wh/kg, see Fig. S3 |
| Number of FOPV plants per pipeline (Npv) | 18.4 | FOPVs | 18.4 | = Qw/Qpv |
| FOPV load factor of pumping energy (η) | 0.114 % |  | 0.114 % | = Ep/(Wfopv･Eas･Esc･Edc) for 300 km, see Fig. 3 |

Abbreviations: floating offshore photovoltaic (FOPV), photovoltaic (PV), ammonia (NH3), liquefied ammonia (LNH3)

Supplementary Table S5. Continental modeling conducted for this study. This table models four continents—Eurasia, North and South America, and Africa—as targets for energy transport in order to assess resource depletion. The modeled continents are rectangular regions extending from the equatorial zone to high-latitude regions.

| Item | Value | Unit | Remarks |
| --- | --- | --- | --- |
| Area of modelled continent (Amc) | 1.27E+08 | km2 | 85.2 % of the total land area |
| Population of modelled continent (Pmc) | 6.44E+09 | persons | 99 % of the world’s population |
| Average population density (Pad) | 50.9 | ersons/km2 | = Pmc/Amc |
| East-west distance of modelled continent (Lew) | 31,675 | km | = Amc/Lns |
| Latitudinal distance of modelled continent (Lns) | 4,000 | km |  |

Supplementary Table S6. Resources for submarine high-voltage direct current (HVDC) cables used for connecting the floating offshore photovoltaic (FOPV) plants and landing stations. The reserves of copper (Cu) and aluminum (Al) required for the manufacture of submarine cables (two cables, one for the positive pole and one for the negative pole) required to transmit the CFE2100 generated in equatorial waters to land from the FOPV plant were evaluated. Note that the power from the FOPV is not smoothed to avoid overloading the FOPV. Consequently, Cu accounted for 19.6% of the resource consumption, whereas Al accounted for 3.3%. Notes: The voltage drop and conductor cross-sectional area calculations were performed using established formulas [49].

| Item | Copper | Aluminum | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Average submarine cable length (Lsma) | 300 | 300 | km | An average cable length was assumed to be 300 km |
| Number of submarine cables (Nsmc) | 51,840 | 51,840 | cables | FOPVs for CFE2100 |
| Maximum transmission voltage (Vsmx) | 500 | 500 | kV |  |
| Maximum transmission current (Ismx) | 9.22 | 9.22 | kA | Transmission without leveling |
| Power transmission loss (Msc) | 5 % | 5 % |  |  |
| Conductor diameter (Dsmc) | 67.8 | 90.1 | mm | Copper is next to sliver, and aluminum is next to copper |
| Submarine cable conductor weight (Wsmc) | 2.12E+04 | 1.04E+04 | tons | Copper and aluminum have specific gravities of 8.96 and 2.71 g/cm3, respectively |
| Total conductor weight (Wtsc) | 1.10E+09 | 5.38E+08 | tons | = Nsmc･Wsmc |
| Conductor reserves [28] (Rcdc) | 5.60E+09 | 1.65E+10 | tons | Estimated copper resources, including undiscovered reserves |
| Total conductor weight/reserves (ξsrc) | 19.6 % | 3.3 % |  | Aluminum is the best conductor for submarine cables |

Abbreviations: floating offshore photovoltaic (FOPV)

Supplementary Table S7. Resources for liquefied ammonia (LNH3) pipelines and high-voltage direct current (HVDC) grids in intracontinental energy transport. This table shows estimates of utilization rates and lifespan of key mineral resources in HVDC and LNH3 transport networks. Each trunk pipeline and HVDC power line was assumed to transport energy from 18.4 FOPV power plants.

| Item | HVDC grid | LNH3 pipeline network | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Average west–east spacing (Swe) | 11.3 | 11.3 | km | = Lew/Npp |
| Average latitudinal spacing (Sns) | 300 | 300 | km | Standard unit length of pipeline transport |
| Total energy transport length (Ltt) | 1.17E+07 | 1.17E+07 | km | = Lew/Swe･Lns+Lns/Sns･Lew |
| Transmission voltage (Vtx) | 500 | － | kV | TES efficiency = 90 %, see Table S10 |
| Average transmission current (Itx) | 31.2 | － | kA | Vtx･Itx･Npv･Etes･(hours/year) = 137 TWh/year |
| LNH3 flow rate (Qw) | － | 47,836 | tons/day | Qw･EDNH3 (= 5,810 Wh/kg)･(days/year) = 101 TWh/year |
| Transmission media | Aluminum | Steel |  |  |
| Transmission unit length (Ltu) | 1,000 | 300 | km |  |
| Energy loss over Ltu (Mtu) | 5.00 % | 0.11 % |  |  |
| Transmission material diameter (Dtm) | 303 | ID: 532 | mm | Pipe OD: 660 mm |
| Material weight over Ltu (Wmu) | 3.90E+05 | 2.84E+05 | tons | Specific gravity: Al = 2.72 g/cm3, Fe = 7.87 g/cm3 |
| Total material weight (Wtm) | 9.10E+09 | 8.56E+09 | tons | = Wmu･Ltt/Ltu×2 ways in HVDC grid |
| Material reserves [28] (Rmt) | 1.63E+10 | 2.30E+11 | tons | Suppose that aluminum reserves are 1/4 of bauxite reserves |
| Resource usage rate (Wtm/Rmt) (Umr) | 56.0 % | 3.7 % |  | LNH3 pipeline can transport carbon-free energy worldwide |
| Total energy loss over Lns (Mel) | 20.0 % | 1.52 % |  | = Mtu･Lns/Ltn |
| Lifespan [47] | 40－50 | 50－60 | years | Proper maintenance is required. The control unit's lifespan is less than half. |
| Energy storage | Impossible | Anywhere |  | HVDC grids can only store energy in LNH3 before converting it into electricity |

Abbreviations: high-voltage direct current (HVDC), liquefied ammonia (LNH3)

Supplementary Table S8. Earth’s energy balance due to the installation of FOPVs. Installing FOPV plants could increase the difference in reflected solar energy (Dsre) between with and without FOPV by 155 PWh. The FOPV plants could reduce the difference in solar energy absorbed by the ocean (Doae) by -367 PWh, cooling the ocean beneath the FOPV model. However, the electricity generated by the FOPV plant (Pbfv) of 416 PWh in 2100 would be released as thermal energy through various processes. Thus, the change in the Earth’s energy balance due to FOPV installation (Eeb) is -106 PWh, resulting in a decrease of -0.024 W/m² in radiative energy globally. This decrease in radiative energy would function to cool the Earth, but the amount would be negligible.

| Parameter | Value | Unit and remarks |
| --- | --- | --- |
| Solar radiation energy to FOPVs in 2100 (Refv) | 2486 | PWh, = Spv･Nfopv･Ravg･Nfv (Nfv in Table 1) |
| Power generation energy of FOPVs in 2100 (Pbfv) | 416 | PWh, = Refv･Sepv/Spv･Epv･Efs･Rop, see Table S1) |
| G-NH3 hydrogen energy in 2100 (Phe) | 285 | PWh, CFE2100 in Table 1 |
| Solar radiation reflected by the FOPVs in 2100 (Rsr) | 404 | PWh, = Refv･((Spv-Sepv)/Spv･65%+Sepv/Spv･8%), see Table1 and Table S1 |
| Solar radiation reflected energy by the ocean without FOPV plant (Rnfv) | 249 | PWh, = Refv･10% |
| Difference in solar reflected energy with and without FOPV plant (Dsre) | -155 | PWh, = Rnfv - Rsr |
| Ocean absorption energy in 2100 with FOPV plant (Oae) | 1870 | PWh, = Refv･92% - Pbfv |
| Ocean absorption energy in 2100 without FOPV plant (Oanpv) | 2237 | PWh, = Refv･90% |
| Difference in ocean absorbed energy with and without FOPV plant (Doae) | -367 | PWh, = Oae - Oanpv |
| Changes in Earth’s energy balance due to FOPV plant installation (Eeb) | -106 | PWh, = Pbfv+Dsre+Doae |
| Decrease in radiant energy at the Earth’s surface by 2100 | -0.024 | W/m2, = Eeb/Esfa/(hours per year), Esfa = 510 million Km2 |

Abbreviations: floating offshore photovoltaic (FOPV)

Supplementary Table S9: Verification of radiative forcing for shared socioeconomic pathways (SSP) scenarios based on the Earth’s energy balance. This table presents the results of a validation study using NASA’s global energy balance data to verify the validity of the numerical analysis model used in this study. The difference between the two was 0.0 °C [−0.1–0.1 °C].

| Parameter | Value | Unit and remarks |
| --- | --- | --- |
| Absorption of solar radiation by Earth’s surface (Ias) | 163.3 | W/m2, see Earth’s energy budget by NASA |
| Back radiation from greenhouse gasses (Ibg) | 340.3 | W/m2, see Earth's energy budget by NASA |
| Emission from the Earth’s surface (Oes) | 398.2 | W/m2, see Earth's energy budget by NASA |
| Thermals (conduction/convection) (Otm) | 18.4 | W/m2, see Earth's energy budget by NASA |
| Latent heat (change of state) (Olh) | 86.4 | W/m2, see Earth’s energy budget by NASA |
| Incoming solar radiation to Earth’s surface (Iic) | 503.6 | W/m2, = Ias+Ibg |
| Outgoing from Earth’s surface (Osf) | 503.0 | W/m2, = Oes+Otm+Olh |
| Difference in incoming and outgoing radiation (Dio) | 0.6 | W/m2, = Iic-Osf |
| Temperature prediction from Dio between 2000 and 2009 (Tdio) | 0.9 [0.6–1.2] | ℃ |
| Average actual surface temperature rise between 2000 and 2009 (Taar) | 0.9 [0.7–1.0] | ℃, calculated by numerical analysis |
| Difference in best estimates of Tdio and Taar (Dioa) | 0.0 [-0.1–0.1] | ℃ |

Supplementary Table S10. Comparison of energy storage methods [39,40]. This table compares the characteristics of energy storage methods used to smooth out significant fluctuations in the output of renewable energy systems: a battery, b CAES, c TES, and d LNH3. Methods a to c are used to smooth out fluctuations in TCE. In contrast, LNH3 has a much higher energy density than a to c, enabling the transport and storage of large amounts of energy over long distances via pipelines.

| Type of energy storage |  | Energy density | Efficiency (%) | Remarks |
| --- | --- | --- | --- | --- |
| (a) Battery | Li-ion secondary | 700 Wh/L 150–250 Wh/kg | 90–95 | Longer (5–10 years or more) lifespan and higher upfront cost. |
|  | lead-acid | 60–110 Wh/L 30–50 Wh/kg | 80–90 | Shorter (3–5 years) lifespan and lower upfront cost. |
| (b) CAES |  | 4–32 Wh/L | 70–80 | Energy is stored in underground caverns and porous layers, enabling grid-scale energy storage. |
| (c) TES |  | 50–600 Wh/L | 70–90 | Stores excess energy and releases it as needed to compensate for the intermittent nature of renewable energy sources. Energy density varies widely depending on the energy storage material. Summer heat can be stored for heating in winter. |
| (d) LNH3 |  | 3,960 Wh/L 5,810 Wh/kg | 74 | As an HEC, it has high energy density. The energy storage material itself can be transported over long distances via pipelines, with little energy loss. See Table S2. |

Abbreviations: Compressed air energy storage (CAES), Liquefied ammonia (LNH3), hydrogen energy carrier (HEC), Thermal energy storage (TES)