# Supplementary Information

# A Study on the Spatio-Temporal Patterns and Driving Mechanisms of Carbon Emissions in the Beijing-Tianjin-Hebei Region from the Perspective of Industrial Relocation

Table S1 Parameter setting for carbon emission estimation of industrial energy

| Energy | NCV(kJ/kg) | CEF(kgC/million kJ) |
| --- | --- | --- |
| raw coal | 20908 | 25.8 |
| coke | 28435 | 29.2 |
| gasoline | 43070 | 18.9 |
| kerosene | 43070 | 19.5 |
| diesel | 42652 | 20.2 |
| fuel oil | 41816 | 21.1 |
| liquefied petroleum gas | 50179 | 17.2 |
| other petroleum products | 9800 | 20.0 |
| natural gas (gaseous) | 38931 | 15.3 |
| washed clean coal | 26344 | 25.8 |
| other coal washing | 8363 | 25.8 |
| crude oil | 41816 | 20.0 |
| refinery dry gas | 46055 | 15.7 |
| coke oven gas | 16726 | 12.1 |
| briquette | 20700 | 26.6 |
| other gases | 15054 | 12.1 |

Table S2 Reference conversion coefficient of industrial energy into standard coal

| Energy | CORC |
| --- | --- |
| coal products | 0.5286kgC/kg |
| producer gas | 0.1786kgC/m3 |
| liquefied natural gas | 1.7572kgC/kg |
| asphalt | 1.3307kgC/kg |
| blast furnace gas | 0.1286kgC/m3 |
| converter gas | 0.2714kgC/m3 |
| coal gangue | 0.2857kgC/kg |
| urban household waste | 0.2714kgC/kg |
| waste heat and pressure | 0.0341tC/106kJ |
| industrial waste | 0.4285kgC/kg |
| bagasse | 0.5000kgC/kg |

Table S3 industrial sectors and industry codes

| Industry code | Industrial sectors |
| --- | --- |
| B | Mining |
| C13 | Processing of Agricultural and Sideline Food Products |
| C14 | Manufacture of Foods |
| C15 | Manufacture of Liquor, Beverages and Refined Tea |
| C17 | Manufacture of Textiles |
| C18 | Manufacture of Textile Wearing Apparel and Ornament |
| C19 | Manufacture of Leather, Fur, Feather and Related Products and Footwear |
| C20 | Processing of Timber, Manufacture of Wood, Bamboo, Rattan, Palm, and Straw Products |
| C21 | Manufacture of Furniture |
| C22 | Manufacture of Paper and Paper Products |
| C23 | Printing and Reproduction of Recording Media |
| C24 | Manufacture of Articles for Culture, Education, Arts and Crafts, Sport and Entertainment Activities |
| C25 | Processing of Petroleum, Coking, and Processing of Nuclear Fuel |
| C26 | Manufacture of Raw Chemical Materials and Chemical Products |
| C27 | Manufacture of Medicines |
| C28 | Manufacture of Chemical Fibers |
| C29 | Manufacture of Rubber and Plastics Products |
| C30 | Manufacture of Non-metallic Mineral Products |
| C31 | Smelting and Pressing of Ferrous Metals |
| C32 | Smelting and Pressing of Non-ferrous Metals |
| C33 | Manufacture of Metal Products |
| C34 | Manufacture of General Purpose Machinery |
| C35 | Manufacture of Special Purpose Machinery |
| C36 | Manufacture of Automobiles |
| C37 | Other Transport Equipment |
| C38 | Manufacture of Electrical Machinery and Apparatus |
| C39 | Manufacture of Computers, Communication and Other Electronic Equipment |
| C40 | Manufacture of Measuring Instruments and Machinery |
| C41 | Other Manufacturing |
| C42 | Comprehensive Utilization of Waste Resources |
| D | Production and Supply of Electricity, Heat, Gas and Water |