Opposing Carbon Dioxide Removals and emissions reductions confuses mitigation policies

Table s1. Analysis of CDR-classified methodologies

| Methodology | Classified as… | Brief description | Active removal from the atmosphere? | Permanent storage? |
| --- | --- | --- | --- | --- |
| VM0047 (ARR): Afforestation, Reforestation & Revegetation, v1.1 (VERRA) | Removal | Considers increases in the density of trees and other types of woody vegetation | Yes, and due to either direct or indirect activities | It depends on fires, harvest, pests, land use policy or any other factors affecting tree health |
| VM0032: Adoption of Sustainable Grasslands through Adjustment of Fire and Grazing, v1.0 (VERRA) | Emissions reduction (methane from animals) and removal (soil carbon) | Sustainable adjustment of the timing/ density of grazing animals and the intensity/ frequency of prescribed fires | Yes, soil sequestration by manipulating variables that determine the grazing activities and fire management in grassland areas | As long as the soils’ dynamics and fire management are maintained |
| VM0042: Improved Agricultural Land Management, v2.1 (VERRA) | Emission reductions and soil organic carbon removal | Improved agricultural land management practices: reduced tillage, improved fertilizer application, etc. | Yes, if woody biomass is included in the project boundary with associated A/R | As long as improved agricultural practices are maintained |
| VM0044: Biochar Utilization in Soil and Non-Soil Applications, v1.2 (VERRA) | Removal | Removals from the conversion of waste biomass into biochar in greenfield facilities | No, as the methodology only accepts biogenic waste biomass and not purpose-grown biomass | The methodology requires soil and non-soil applications that “prove carbon-persistent over time” |
| VM0033: Tidal Wetland and Seagrass Restoration, v2.1 (VERRA) | Emissions reduction (e.g. reduced peat depletion, forest or soil subsidence) and removal (e.g. increased vegetation cover) | Activities aimed at restoring and creating tidal wetlands: managing hydrological conditions, sediment supply, etc. | Yes, by increasing biomass production | As long as sustainable management of tidal wetlands and seagrass is maintained |
| VM0049: Carbon Capture and Storage, v1.0 (VERRA) | Emission reductions and removal | Project activities capture CO2 from the atmosphere or point sources at a facility and store it permanently in geological storage complexes | Only in the case of Direct Air Capture, but the methodology also includes post-combustion and pre-combustion capture (among others) that are not associated with an active removal of carbon | Storage is assumed permanent/ very long term |
| Carbonated Materials Methodology for CO2 Removal (puro.earth) | Removal | To be counted as removal, the CO2 mineralised in the carbonated material should be of biogenic origin or from direct capture from the ambient atmosphere | Not necessarily. As the criterion for including carbon of “biogenic origin” is “of eligible types, and EU or national legislation must be followed in the sourcing and extraction of the raw materials used”, we conclude that the scope is wide and includes, e.g. residues. Furthermore, CO2 removal can be achieved by “preventing its entrance in the atmosphere and converting the CO2 to a stabile, durable storage”. | Yes, according to the assessment of carbonated material by the IPCC |
| Terrestrial storage of biomass (puro.earth) | Removal | It quantifies the net CO2 removal achieved over 100 years by the storage of eligible biomass (lignocellulosic: trees and hard-stemmed plants) in adequate terrestrial storage systems | Not necessarily as eligible biomass includes waste biomass with carbon capture outside of the project boundary | It depends on the storage facilities and conditions, but assumed to be long-term as long as the storage place is not disturbed (e.g. water infiltrations or outright destruction) |
| Enhanced Rock Weathering (puro.earth) | Removal | It aims to accelerate the chemical weathering process by spreading finely ground rock powder or other suitable weathering materials to the soil | Yes, due to the specific and active establishment of the conditions (e.g. material spreading to the soil) for enhanced rock weathering to happen | Yes, it is documented as one of the most permanent forms of removal with low risk of reversal |
| Ocean alkalinity enhancement (OAE) from coastal outfalls (Isometric) | Removal (and avoided emissions?) | OAE aims to speed up natural processes by adding CO₂-reactive alkalinity to the surface ocean, leading to re-equilibration of the atmospheric and oceanic carbon reservoirs through increasing the flux of CO₂ from the atmosphere to the ocean, or reducing the outgassing of CO₂ | Yes because the action of enhancing alkalinity in the ocean leads to more intake of CO2 by the oceans; but enhanced alkalinity could also reduce outgassing of CO2 from the oceans, which is equivalent to avoided emissions | In theory removals are very long term (10 k- 100 k years), but reversal can occur due to large-scale deployment of CDR, and/ or reduced concentration of CO2 in the atmosphere resulting in outgassing from the ocean |
| Biomass fermentation with carbon capture and geological storage (Gold Standard) | Removal | CO₂ from biomass fermentation is captured and stored in geological reservoirs; the source of CO₂ must be renewable biomass (“either cultivated in dedicated plantations or is derived from biomass residues”) | Not necessarily, it depends on the source of biomass; as eligible biomass can be residues, then cases of avoided emissions are included | Storage is assumed permanent/ very long term (geological reservoirs) |
| Bioenergy with carbon capture and storage (BECSS) (Drax & Stockholm Exergi) | Removal | BECCS technology captures CO₂ and permanently stores it in geological reservoirs, and CO₂ must come from “sustainably sourced biomass”; it applies to BECCS projects using thermal combustion of solid biomass fuels | Depends on the cases, but unlikely so; when the project includes the retrofitting of an existing biomass power and/ or heat plant there is no active removal of carbon because the biomass would have been produced and supplied regardless | Storage is assumed permanent/ very long term (geological reservoirs) |