# From Construction Burden to Operational Benefits: A Life Cycle Analysis of Sponge City Source Control Facilities

Changqing Xua,b,1, Yuewen Luob,1, Tianyu Jiab*, Hua Chai c,d*, Haifeng Jiae,*

a Beijing Key Laboratory of Urban Hydrological Cycle and Sponge City Technology, Beijing 100875, China

b School of Economics, Beijing Institute of Technology, Beijing 100081, China

c State Key Laboratory of Urban and Regional Ecology, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing, 100085, China.

d University of the Chinese Academy of Sciences, Beijing, 100049, China

e School of Environment, Tsinghua University, Beijing 100084, China

1 These authors contributed equally.

* Corresponding author: 3120246164@bit.edu.cn; chaihua23@mails.ucas.ac.cn; jhf@tsinghua.edu.cn

The supporting information contains 6 pages, 4 tables.

# Supplementary information

Tab. S1 The values from U1 to U6

|  |  | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| U1 | Reliability | 1.00 | 1.05 | 1.10 | 1.20 | 1.50 |
| U2 | Completeness | 1.00 | 1.02 | 1.05 | 1.10 | 1.20 |
| U3 | Temporal correlation | 1.00 | 1.03 | 1.10 | 1.20 | 1.50 |
| U4 | Geographical correlation | 1.00 | 1.01 | 1.02 | 1.05 | 1.10 |
| U5 | Further technological correlation | 1.00 | 1.10 | 1.20 | 1.50 | 2.00 |
| U6 | Sample size | 1.00 | 1.02 | 1.05 | 1.10 | 1.20 |

Tab. S2 Monte Carlo simulation Ub value table

|  | Input/Output group | a | p | a |
| --- | --- | --- | --- | --- |
| Demand of | thermal energy, electricity, semi-finished products, working materials, waste treatment services | 1.05 | 1.05 | 1.05 |
|  | transport services (tkm) | 2.00 | 2.00 | 2.00 |
|  | infrastructure | 3.00 | 3.00 | 3.00 |
| Resources | primary energy carriers, metals, salts | 1.05 | 1.05 | 1.05 |
|  | land use, occupation | 1.50 | 1.50 | 1.50 |
|  | land use, transformation | 2.00 | 2.00 | 2.00 |
| Pollutants emitted to water | BOD, COD, DOC, TOC, inorganic compounds (NH4, PO4, NO3, Cl, Na, etc.) |  | 1.50 |  |
|  | individual hydrocarbons, PAH |  | 3.00 |  |
|  | heavy metals |  | 5.00 | 1.80 |
|  | pesticides |  |  | 1.50 |
|  | NO3, PO4 |  |  | 1.50 |
| Pollutants emitted to soil | oil, hydrocarbon total |  | 1.50 |  |
|  | heavy metals |  | 1.50 | 1.50 |
|  | pesticides |  |  | 1.20 |
| Pollutants emitted to air | CO2 | 1.05 | 1.05 |  |
|  | SO2 | 1.05 |  |  |
|  | NMVOC total | 1.50 |  |  |
|  | Nox, N2O | 1.50 |  | 1.40 |
|  | CH4,NH3 | 1.50 |  | 1.20 |
|  | individual hydrocarbons | 1.50 | 2.00 |  |
|  | PM>10 | 1.50 | 1.50 |  |
|  | PM10 | 2.00 | 2.00 |  |
|  | PM2.5 | 3.00 | 3.00 |  |
|  | polycyclic aromatic hydrocarbon(PAH) | 3.00 |  |  |
|  | CO, heavy metals | 5.00 |  |  |
|  | inorganic emissions, others |  | 1.50 |  |
|  | radionuclides (eg. Radon-222) |  | 3.00 |  |

Tab. S3 LCIA characterization results for SCSCFs and the contribution of main process to in the Recipe mid-point scores during construction phase

| Categories | Unit |  | Grassed swales |  |  | Bioretention cells |  |  |  | Infiltration pits |  |  |  | Buffer strips |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Value |  | Process | Value |  |  | Process | Value |  |  | Process | Value |  |  | Process |
| Global warming |  | kg CO2 eq |  | 2.05×103 | Concrete (51%) +Transportation (41%) |  | 4.17×103 | Non-woven fabrics（50%） +PVC（25%） +Transportation（25%） |  |  | 1.40×104 | PVC(40%) +Concrete (55%) |  |  | 1.37×103 | PVC(40%) +Concrete(55%) |  |
| Stratospheric ozone depletion |  | kg CFC11 eq |  | 0.00 | Non-woven fabrics (49%) +Transportation (33%) |  | 0.02 | Non-woven fabrics（92%） |  |  | 0.01 | PVC(34%) +Non-woven fabrics（37%） +Concrete (29%) |  |  | 0.00 | PVC(85%) |  |
| Ionizing radiation |  | kBq Co-60 eq |  | 31.40 | Concrete (27%) +Transportation (48%) |  | 1.57×102 | Non-woven fabrics（52%） +PVC（36%） |  |  | 4.65×102 | PVC(64%) +Concrete (30%) |  |  | 59.91 | PVC(94%) |  |
| Ozone formation, Human health |  | kg NOx eq |  | 7.60 | Concrete (44%) +Transportation (50%) |  | 14.01 | Non-woven fabrics（49%） +Transportation（33%） |  |  | 50.03 | PVC(25%) +Concrete (70%) |  |  | 3.47 | PVC(69%) +Concrete(23%) |  |
| Fine particulate matter formation |  | kg PM2.5 eq |  | 2.95 | Concrete(54%) +Transportation (35%) |  | 7.72 | Non-woven fabrics（63%） +PVC（20%） |  |  | 19.48 | PVC(42%) +Concrete (50%) |  |  | 2.03 | PVC(77%) +Concrete(19%) |  |
| Ozone formation, Terrestrial ecosystems |  | kg NOx eq |  | 7.71 | Concrete(44%) +Transportation (50%) |  | 14.26 | Non-woven fabrics（49%） +Transportation（33%） |  |  | 51.18 | PVC(26%) +Concrete (70%) |  |  | 3.57 | PVC(70%) +Concrete(23%) |  |
| Terrestrial acidification |  | kg SO2 eq |  | 8.39 | Concrete(58%) +Transportation (30%) |  | 25.40 | Non-woven fabrics（75%） |  |  | 46.82 | PVC(38%) +Concrete (49%) |  |  | 4.80 | PVC(70%) +Concrete(25%) |  |
| Freshwater eutrophication |  | kg P eq |  | 0.47 | Concrete(42%) +Transportation (35%) |  | 2.62 | Non-woven fabrics（78%） |  |  | 4.10 | PVC(46%) +Concrete (38%) |  |  | 0.43 | PVC(84%) |  |
| Marine eutrophication |  | kg N eq |  | 0.49 | Non-woven fabrics(96%) |  | 12.17 | Non-woven fabrics（100%） |  |  | 4.06 | Non-woven fabrics（93%） |  |  | 0.11 | PVC(38%) +Non-woven fabrics（60%） |  |
| Terrestrial ecotoxicity |  | kg 1,4-DCB |  | 1.76×104 | Transportation (83%) |  | 2.90×104 | Transportation（62%） +Non-woven fabrics（20%） |  |  | 1.65×105 | Concrete (83%) |  |  | 6.50×103 | PVC(78%） |  |
| Freshwater ecotoxicity |  | kg 1,4-DCB |  | 44.62 | Concrete(40%) +Transportation (37%) |  | 2.33×102 | Non-woven fabrics（70%） +PVC（20%） |  |  | 4.72×102 | PVC(57%) +Concrete (32%) |  |  | 57.02 | PVC(89%) |  |
| Marine ecotoxicity |  | kg 1,4-DCB |  | 64.98 | Concrete(37%) +Transportation (46%) |  | 2.50×102 | Non-woven fabrics（59%） +PVC（27%） |  |  | 6.77×102 | PVC(52%) +Concrete (41%) |  |  | 75.62 | PVC(89%) |  |
| Human carcinogenic toxicity |  | kg 1,4-DCB |  | 70.05 | Concrete(40%) +Transportation (48%) |  | 1.85×102 | Non-woven fabrics（45%） +PVC（33%） +Transportation（23%） |  |  | 6.60×102 | PVC(48%) +Concrete (48%) |  |  | 69.65 | PVC(87%) |  |
| Human non-carcinogenic toxicity |  | kg 1,4-DCB |  | 1.19×103 | Concrete(35%) +Transportation (48%) |  | 4.59×103 | Non-woven fabrics（62%） +PVC（22%） |  |  | 1.17×104 | PVC(46%) +Concrete (46%) |  |  | 1.18×103 | PVC(87%) |  |
| Land use |  | m2a crop eq |  | 1.42×102 | Non-woven fabrics（45%） +Concrete(23%) +Transportation (31%) |  | 1.73×104 | Non-woven fabrics（96%） |  |  | 1.03×103 | Non-woven fabrics(50%) +Concrete (39%) |  |  | 40.50 | PVC(52%) +Non-woven fabrics(22%) +Concrete（20%） |  |
| Mineral resource scarcity |  | kg Cu eq |  | 5.96 | Concrete(63%) +Transportation (28%) |  | 11.47 | Non-woven fabrics（49%） +PVC（33%） |  |  | 37.46 | PVC(54%) +Concrete (42%) |  |  | 4.85 | PVC(79%) +Concrete(19%) |  |
| Fossil resource scarcity |  | kg oil eq |  | 4.89×102 | Concrete(30%) +Transportation (58%) |  | 1.28×103 | Non-woven fabrics（33%） +PVC（39%） +Transportation（28%） |  |  | 5.41×103 | PVC(48%) +Concrete (49%) |  |  | 5.51×102 | PVC(90%) |  |
| Water consumption |  | m3 |  | 67.81 | Non-woven fabrics（72%） +Concrete(25%) |  | 1.27×103 | Non-woven fabrics（99%） |  |  | 4.80×102 | Non-woven fabrics（82%） |  |  | 25.22 | PVC(57%) +Non-woven fabrics(27%) +Concrete（16%） |  |

Tab. S4 Four SCSCFs cost estimates

(a) Construction cost estimates

| Construction facilities |  | Construction engineering cost | Installation engineering cost | Other construction costs | Basic reserve fund |
| --- | --- | --- | --- | --- | --- |
| Grassed swales | Index base price | 1.72×104 | / | / | / |
| Bioretention cells | Index base price | 3.56×104 | 1.87×103 | / | / |
| Infiltration pits | Index base price | 3.32×104 | 3.32×102 | / | / |
| Buffer strips | Index base price | 5.12×104 | / | 3.46×103 | 2.40×103 |

(b) O&M cost estimates

| Construction facilities | Water consumption | Water price | Fertilizer application amount | Organic fertilizer price | Maintenance personnel | Labor cost |
| --- | --- | --- | --- | --- | --- | --- |
| Grassed swales | 2.71×103 | 5 | 3.87×103 | 0.5 | 1 | 150 |
| Bioretention cells | 1.43×103 | 5 | 7.82×102 | 0.5 | 1 | 150 |
| Infiltration pits | 4.35×103 | 5 | 1.55×103 | 0.5 | 2 | 150 |
| Buffer strips | 2.50×103 | 5 | 2.38×102 | 0.5 | 1 | 150 |

(c) Disposal cost estimates

| Construction facilities | Excavation cost | Transportation cost (30km) |
| --- | --- | --- |
| Grassed swales | 9.28×103 | 7.43×103 |
| Bioretention cells | 4.89×103 | 3.91×103 |
| Infiltration pits | 2.65×103 | 2.12×103 |
| Buffer strips | 7.33×103 | 5.86×103 |