Supplementary material

Spatial indicators used for CoolCity Index (CCI) calculation, their source of information (raster map), definitions, weight in integrated indicator and inclusion in the integrated indicators.

| Spatial indicators | Raster maps (Source of information) | Description | The weight in integrated indicators | Inclusion in the integrated indicator |
| --- | --- | --- | --- | --- |
| Vegetation cover | Land cover map | Percentage of vegetation in the city | 0.4 | Vegetation state |
| High vegetation cover | Land cover map | Percentage of high vegetation (forests and dense tree cover) in the city | 0.5 |  |
| Share of high-vigor vegetation | Land cover map vegetation vigor map | Percentage of vegetation (excluding arable land) with high vigor in the city | 0.1 |  |
| Surface radiation properties | Surface radiation properties map | 95th percentile of surface radiation property values within the city | 0.2 | Thermal conditions |
| Surface temperature | Land surface temperature map | City surface temperature expressed in °C | 0.6 |  |
| Share of high-temperature areas | Land surface temperature map | Percentage of high-temperature areas in the city. For each city, the mean LST is first calculated. Next, the 10 nearest neighboring cities are identified, and the mean LST of these neighboring cities is computed. The temperature threshold is then defined as: Tthr=T10 nearest neighbors + 2σcity, where T10 nearest neighbors​ denotes the average LST of the 10 nearest neighboring cities, and σcity​ represents the standard deviation of pixel-level temperatures within the analyzed city. Pixels with temperatures exceeding this threshold are classified as high-temperature areas. The final indicator is expressed as the percentage of the city area covered by these pixels relative to the total city area. | 0.1 |  |
| Share of low-temperature areas | Land surface temperature map | Percentage of low-temperature areas in the city. For each city, the mean LST is first calculated. Next, the 10 nearest neighboring cities are identified, and the mean LST of these neighboring cities is computed. The temperature threshold is then defined as: Tthr=T10 nearest neighbors - 2σcity, where T10 nearest neighbors​ denotes the average LST of the 10 nearest neighboring cities, and σcity​ represents the standard deviation of pixel-level temperatures within the analyzed city. Pixels with temperatures exceeding this threshold are classified as high-temperature areas. The final indicator is expressed as the percentage of the city area covered by these pixels relative to the total city area. | 0.1 |  |
| Water retention potential | Land cover map | Sum of retention coefficients for the land cover classes present in the city | 0.6 | Water conditions |
| Share of wetland ecosystems | Land cover map | Percentage of the city’s area covered by water bodies and wetlands | 0.4 |  |
| Biologically active area | Land cover map | Percentage of areas covered by biologically active surfaces in the city | 1.0 | Surface permeability |
| Ecological connectivity | Land cover map | Degree of connectivity between vegetation patches in the city. Methodology based on Indicator 2: Connectivity Measures or Ecological Networks to Counter Fragmentation from the Singapore Index on Cities’ Biodiversity (Chan et al. 2021) | 0.2 | Vegetation diversity |
| Vegetation diversity | Land cover map | The Gini–Simpson diversity index (Simpson 1949) represents the diversity of vegetation classes in the city. It is calculated using the area and number of patches of each vegetation class within a hexagonal grid with a side length of approximately 200 m | 0.1 |  |
| Vegetation richness | Land cover map | The average number of vegetation classes within a hexagonal grid with a side length of approximately 200 m | 0.5 |  |
| High biodiversity | Land cover map | Percentage of land cover classes in the city with potentially high biodiversity | 0.2 |  |