# SUPPLEMENTARY INFORMATION 2

# Setting corporate climate targets based on the consumer needs companies support

# Teddy Serrano1,2,*, Moritz Günther3, Anders Bjørn1,2, Michael Hauschild1,2

1Centre for Absolute Sustainability, Technical University of Denmark, Kgs Lyngby, Denmark

2Section for Quantitative Sustainability Assessment, Department of Environmental and Resource Engineering, Technical University of Denmark, Kgs Lyngby, Denmark

3Department Sustainability Management and Life Cycle Engineering, Fraunhofer Institute for Surface Engineering and Thin Films IST, Braunschweig, Germany

*Corresponding author: tadse@dtu.dk

## Completeness proof of the approach

This section presents the completeness proof showing that a climate sustainability score of 1 of all companies guarantees global emissions stay within the defined carrying capacity. To keep the demonstration straightforward, the proof considers a simplified case with only one societal function worldwide and excludes emissions from the products’ use phase.

Let S be the emissions matrix, and Si,j be the direct emissions of company j to make the consumer product i.

We denote Si,*= jSi,j the total emissions from the product i’s life cycle, assuming that no direct emissions from consumers are occurring, and S*,j= iSi,jthe emissions of company j. We also denote  S*=iSi,*=jS*,j

We denote fi the quantity of functional units fulfilled by product i. We denote F= ifi, the total quantity of functional units fulfilled by all products. We finally denote L the global emissions limit.

The climate sustainability score of company j, written CSSj, is defined as:

CSSj=iαi,j⋅xiαi,j= Si,jkSk,jxi=Si,*⋅FL⋅fi

The condition that ensures that companies being sustainable in absolute terms implies that the world’s emissions are below the global limit is:

∀j, CSSj≤1 ⟹S*≤L

Let’s assume:

∀j, CSSj≤1

Then:
iSi,jkSk,j⋅xi≤1

⟹ iSi,j⋅xi≤iSi,j

⟹ jiSi,j⋅xi≤jiSi,j

⟹ ijSi,j⋅xi≤ijSi,j

⟹ ixi⋅Si,*≤S*

⟹ iSi,*2⋅FL⋅fi≤S*

⟹ iSi,*2fi≤L⋅S*F

Using Cauchy-Schwarz inequality:

⟹ (iSi,*)2ifi≤iSi,*2fi≤L⋅S*F

⟹ S*2F≤L⋅S*F

S*≤L

Mathematically, this inequality becomes an equality when all consumer products have the same life cyle emissions intensities, i.e.:

∃λ, ∀i, Si,*fi= λ