Supplementary Information

for

Three-year field performance of a residential grid-tied PV prosumer in Romania: measured data, seasonal yield, and statistical characterisation

George-Andrei Beșchea, Beatrice-Georgiana Vuțoiu*, Gabriel Năstase

Laboratory of Building Services, Institute of Civil Engineering, Transilvania University of Brașov, 500152 Brașov, Romania

* Corresponding author. E-mail: beatrice.vutoiu@unitbv.ro

Contents

Supplementary Table S1. Inferred installed capacity sensitivity analysis.

Supplementary Table S2. Study limitations and recommended mitigations.

Supplementary Table S3. Descriptive statistics of monthly production data.

Supplementary Table S4. List of abbreviations.

Supplementary Table S1

Specific yield in kWh/kWp/yr computed under eight candidate rated capacities, compared to the PVGIS-SARAH3 expected range of 1,050–1,150 kWh/kWp/yr for central Romania at optimal fixed tilt (reference 3 in the main manuscript). The highlighted row (2.70 kWp) indicates the candidate capacity for which the three-year mean specific yield falls within the PVGIS expected range. This table supports the inferred installed-capacity discussion in Section 3.4.2 of the main text.

| kWp | 2023 | 2024 | 2025 | 3-yr avg | Δ from PVGIS range |
| --- | --- | --- | --- | --- | --- |
| 2.50 | 1,278.0 | 1,268.5 | 1,153.5 | 1,233.3 | +7.2% above |
| 2.70 | 1,183.3 | 1,174.6 | 1,068.0 | 1,142.0 | ✓ within range |
| 3.00 | 1,065.0 | 1,057.1 | 961.2 | 1,027.8 | −2.1% below |
| 3.25 | 983.1 | 975.8 | 887.3 | 948.7 | −9.6% below |
| 3.50 | 912.9 | 906.1 | 823.9 | 881.0 | −16.1% below |
| 4.00 | 798.7 | 792.8 | 720.9 | 770.8 | −26.6% below |
| 4.50 | 710.0 | 704.7 | 640.8 | 685.2 | −34.7% below |
| 5.00 | 639.0 | 634.3 | 576.7 | 616.7 | −41.3% below |

Supplementary Table S2

Systematic inventory of study limitations together with recommended mitigations. This table supports the discussion of limitations in Section 4.3 of the main text.

| # | Limitation | Impact on study | Recommended mitigation |
| --- | --- | --- | --- |
| 1 | Rated peak power (kWp) unknown | Precludes verified specific yield & PR calculation per IEC 61724 | Retrieve from ATR documentation; add to monitoring system |
| 2 | No on-site irradiance reference | Cannot distinguish irradiance variability from inverter degradation | Install a calibrated pyranometer (Class C or better) |
| 3 | Household load not sub-metered | Consumption profile unknown; cannot assess time-of-use optimisation | Install sub-metering on major load categories |
| 4 | Data source: mobile app screenshots | Manual extraction risk; no automated API export | Configure ShineServer API automated export to CSV/JSON |
| 5 | Monitoring period: 37 months | Below recommended 5–10 yr for degradation trend analysis | Continue data collection; re-evaluate at Year 5 (2027) |
| 6 | Single installation (n=1) | Results may not generalise to broader population | Replicate study with ≥20 similar installations in the same region |

Supplementary Table S3

Descriptive statistics computed from the 12 monthly production values for each year. Q1 and Q3 refer to the first and third quartiles of the monthly distribution, not to meteorological quarters. IQR = interquartile range.

| Statistic | 2023 | 2024 | 2025 |
| --- | --- | --- | --- |
| N (months) | 12 | 12 | 12 |
| Mean (kWh/month) | 266.3 | 264.3 | 240.3 |
| Median (kWh/month) | 272.5 | 268.6 | 246.7 |
| Std. Dev. (kWh) | 126.5 | 117.5 | 95.8 |
| Coeff. of Variation | 47.5% | 44.5% | 39.9% |
| Minimum (kWh) | 97.7 (Dec) | 57.9 (Dec) | 89.6 (Dec) |
| Maximum (kWh) | 420.7 (Jul) | 419.5 (Jun) | 377.2 (Jun) |
| Range (kWh) | 323.0 | 361.6 | 287.6 |
| Q1 Quartile (kWh) | 105.6 | 104.1 | 112.7 |
| Q3 Quartile (kWh) | 395.5 | 379.5 | 343.1 |
| IQR (kWh) | 289.9 | 275.4 | 230.4 |
| Annual Total (kWh) | 3,195.0 | 3,171.3 | 2,883.7 |

The coefficient of variation declines monotonically from 47.5% in 2023 to 44.5% in 2024 and 39.9% in 2025. The trend reflects the changes observed in the seasonal production profile: in 2025, simultaneously higher winter production and lower summer production relative to 2023 compressed the seasonal amplitude and brought the monthly production distribution closer to uniformity.

Supplementary Table S4

List of abbreviations used in the main manuscript.

| Abbreviation | Full term |
| --- | --- |
| ANRE | Autoritatea Națională de Reglementare în domeniul Energiei (Romanian Energy Regulatory Authority) |
| ATR | Aviz Tehnic de Racordare (Technical Approval for Grid Connection) |
| CAMS | Copernicus Atmosphere Monitoring Service |
| ECMWF | European Centre for Medium-Range Weather Forecasts |
| GHI | Global Horizontal Irradiance |
| IEA | International Energy Agency |
| IEC | International Electrotechnical Commission |
| MPPT | Maximum Power Point Tracking |
| NAO | North Atlantic Oscillation |
| POA | Plane of Array |
| PR | Performance Ratio |
| PV | Photovoltaic |
| PVGIS | Photovoltaic Geographical Information System (JRC, European Commission) |
| SCF | Self-Consumption Fraction |
| STL | Seasonal-Trend decomposition using Loess |