# Supplementary Information for

# Adsorption and separation of U(VI) by hafnium phosphate modified with phosphomuscle alcohol

Youke Shi1, Youqun Wang1,2*, Rongxi Hu2, Zhibin Zhang1, Xiaohong Cao1*, Yunhai Liu 1,3

1 National Key Laboratory of Prospecting, Mining and Remote Sense Detecting on Uranium Resources, East China University of Technology, Nanchang, Jiangxi 330013, China.

2 School of Nuclear Science and Engineering, East China University of Technology, Nanchang, 330013 China.

3 Gandong University, Fuzhou, Jiangxi 344000, China.

The U(VI) amount adsorbed (q, mg/g) and distribution ratio (kd, g/mL) was calculated by Eq. 1 and 2, respectively[16-18].

| qe=(c0-ce)×Vm | (s1) |
| --- | --- |
| kd=qece | (s2) |

Where c0 and ce represent the initial and equilibrium concentrations of uranium, respectively, in mg/L. m and V stands for the mass (g) of the adsorbent and the volume (L) of the solution, respectively.

qt=qe(1-e-k1t)                                (s3)

qe=k2qe2t 1+k2qet                              (s4)

Where k1(min-1) and k2(g·mg-1·min-1) represent the rate constants of pseudo-first-order and pseudo-second-order adsorption kinetics, respectively. Here, qe represents the equilibrium adsorption amount, qt is the adsorption amount at time t.

Qe=kFCe1/n                              (s5)

qe=kLqm ce1+kLce                              (s6)

Where qm is the maximum adsorption capacity, mg/g, KL is the Langmuir adsorption constant, L/mg, and KF and n are the Freundlich adsorption constants, respectively.

（s7）

（s8）

Among them, T is the temperature of the reaction; R is the gas molar constant, which is 8.314 J·mol-1·K-1, and Kd is the solution partition coefficient.

Corresponding author. Tel: +86 791 83897791.

E-mail address: wangyouqun@ecut.edu.cn (Y. Wang), xhcao@ecut.edu.cn (X. Cao).