MRI Acquisition Protocol:

The acquisition was performed using 1.5 Tesla scanners, specifically a Signa scanner manufactured by GE Healthcare, Milwaukee, WI. A bilateral phased-array breast coil was used as the receiver. The MR imaging protocol included a 3D localizer and a unilateral sagittal DCE acquisition. The DCE acquisition utilized a high spatial resolution, low temporal resolution, T1-weighted, fat-suppressed 3D fast gradient-recalled echo sequence developed for pre-surgical staging. The sequence parameters were: TR/TE 8/4.2 milliseconds, flip angle 20 degrees, field of view 18-20 cm, acquisition matrix 256 x 192 x 60, section thickness 2 mm, with spatial resolution 0.7 x 0.94 x 2.0 mm. The temporal phases included pre-contrast (t0) and two post-contrast phases: t1 at approximately 2.5 minutes and t2 at approximately 7.5 minutes. The contrast agent was gadopentetate dimeglumine (Magnevist, Bayer HealthCare, Berlin, Germany) at 0.1 mmol/kg body weight injected at 1.2 mL/s followed by 10 mL saline flush. Fat suppression was performed using a frequency-selective inversion recovery preparatory pulse. Total acquisition time was approximately 5 minutes per phase. All sites submitted raw DICOM series, subtraction images and available diffusion sequences; only the primary DCE series were used for quantitative analysis. DICOM series were imported into an in-house Python 3.9 toolset replicating the UCSF SER-FTV algorithm. Datasets were re-oriented to a common sagittal coordinate frame; consensus resampling to 0.8 mm isotropic voxels using B-spline interpolation ensured voxel-wise correspondence across time points. Rigid body registration with six degrees-of-freedom aligned post-contrast phases to the pre-contrast volume via mutual-information minimization, correcting for between-phase patient motion.

MRI data were analyzed to quantify tumor burden and enhancement dynamics using a semi-automated post-processing tool developed in the IDL programming environment (ITT Visual Information Solutions, Boulder, CO). An experienced radiologist manually defined a three-dimensional volume of interest (VOI) for each tumor by placing rectangular regions of interest (ROIs) on two orthogonal maximum-intensity-projection (MIP) images from the early post-contrast series. The intersection of these ROIs was used to delineate the VOI, capturing all visibly enhancing tumor tissue. Non-malignant enhancing areas (e.g., vessels or cardiac structures) included within the VOI were excluded manually using freehand irregular ROIs. Following this initial segmentation, all subsequent analyses were fully automated.

At every voxel inside the VOI, Early Percent Enhancement was calculated as PE = 100 × (S₁ − S₀)/S₀, where S₀, S₁, S₂ denote pre-contrast, early and late phase signal intensities respectively. Signal-Enhancement Ratio was calculated as SER = (S₁ − S₀)/(S₂ − S₀). Voxels were retained as "functionally active tumor" if they satisfied: PE ≥ 70% (site-adjusted threshold validated in ACRIN 6657); SER > 0 (indicative of wash-out or plateau kinetics); pre-contrast intensity ≥ 60% of VOI maximum (excludes suppressed fat); and connectivity to ≥ 4 adjacent PE-positive voxels in 3D, minimizing noise-driven speckles. The aggregate volume of retained voxels (converted from voxel count × voxel volume) yielded SER-defined FTV (cm³). For each scan, morphologic longest diameter was measured on the early phase and whole tumor volume was calculated via threshold-based region-growing at PE ≥ 50% irrespective of SER.