Table 2: Biomechanical characteristics during the picking-up task between the good and poor balance and mobility groups.

| Characteristic | Picking up taska, b, c |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | P1 |  | P2 |  | P3 |  |
|  | Good (n = 91) | Poor (n = 44) | Good (n = 91) | Poor (n = 44) | Good (n = 91) | Poor (n = 44) |
| C7 height (cm/cmBH) | 0.64 ± 0.06 | 0.71 ± 0.06* | 0.52 ± 0.02 | 0.54 ± 0.03 | 0.81 ± 0.04 | 0.79 ± 0.05 |
| Left squatting angle (degree) | 13.88 ± 2.97 | 11.09 ± 2.85* | 16.53 ± 3.59 | 14.56 ± 4.16 | 8.04 ± 3.01 | 8.48 ± 2.93 |
| Right squatting angle (degree) | 17.69 ± 3.24 | 14.27 ± 3.37* | 19.32 ± 3.49 | 17.13 ± 4.29 | 9.32 ± 3.11 | 9.64 ± 3.04 |
| Right eccentric hip extensor power (W/kg*m) | 0.89 ± 0.39 | 0.53 ± 0.38* | -0.74 ± 0.39 | -0.57 ± 0.62 | -0.27 ± 0.24 | -0.32 ± 0.36 |
| Vertical acceleration of left acromion (cm/s2) | 94.29 ± 111.21 | 22.46 ± 76.31 | 330.95 ± 104.87 | 232.65 ± 96.12 | -309.61 ± 154.32 | -194.41 ± 129.47 |
| Vertical acceleration of right acromion (cm/s2) | 120.75 ± 132.15 | 29.76 ± 87.93 | 393.76 ± 138.92 | 264.85 ± 118.95† | -338.40 ± 160.71 | -207.71 ± 136.39 |
| Vertical acceleration of C7 (cm/s2) | 99.07 ± 117.10 | 16.70 ± 75.69 | 364.46 ± 115.63 | 246.26 ± 104.88† | -333.72 ± 154.92 | -208.24 ± 132.37 |
| Vertical acceleration of T12 (cm/s2) | 8.58 ± 100.43 | -30.85 ± 54.17 | 220.95 ± 89.74 | 151.81 ± 82.19 | -213.91 ± 129.77 | -146.05 ± 92.6 |
| Vertical acceleration of right elbow (cm/s2) | 118.97 ± 141.13 | 15.81 ± 99.39 | 389.59 ± 141.62 | 268.03 ± 122.37 | -326.97 ± 155.74 | -215.29 ± 121.57 |
| Vertical acceleration of right wrist (cm/s2) | 120.61 ± 149.57 | 26.16 ± 95.08 | 382.12 ± 139.21 | 281.23 ± 123.37 | -320.83 ± 154.82 | -213.86 ± 134.39 |
| Vertical acceleration of center of mass (cm/s2) | 53.02 ± 77.25 | 7.11 ± 42.56 | 212.95 ± 79.77 | 148.13 ± 69.06 | -191.41 ± 113.68 | -117.92 ± 88.38 |

The data are presented as the means ± standard deviations. Anatomical landmark height data were normalized to body height, whereas joint power was normalized to body weight multiplied by body height. aIndependent sample t test, bMann-Whitney U test, and canalysis of covariance. cmBH = cm of body height, kgBW = kg of body weight, P1 = the midpoint of the reaching-down phase, P2 = the start of the lifting phase, and P3 = the midpoint of the lifting phase. Bonferroni correction was applied for 1,005 between-group comparisons across P1–P3, with statistical significance set at p < 0.00005, *p <0.00005 compared with the good balance and mobility groups at P1. †p <0.00005 compared with the good balance and mobility groups at P2.